TABLE OF CONTENTS

No table of contents entries found.

# Open Source FEA using adapy

This report describes the main verification work of the simulation results produced by the Open Source Finite Element (FE) solvers Code Aster and Calculix.

The motivation behind producing this report is to increase confidence in the conversion of FEM models in adapy and also the results from the various FEA solvers.

## Introduction

The results from Code Aster and Calculix and tools will be compared with proprietary FEA solvers such as;

- Abaqus

- Sestra (part of DNV’s Sesam suite)

The simulations are pre- and post processed using adapy and this report is automatically generated using paradoc as part of a ci/cd pipeline within the adapy repositories.

For each new addition of code affecting the handling of FEM in adapy a new set of analyses is performed, whereas the results are gathered in this auto-generated document.

Please note! This report will not go into detail into the specifics of why the results are different between different element representations. This report will only investigate and compare the results from different FEA software.

## FEA Solvers

- Calculix v2.23

- Code Aster v18.1.7

- Abaqus v2025

- Sestra v11.3.1.0

## Python packages

The following python packages were instrumental in the creation of this document and the FEA results herein.

### Adapy

The intention behind adapy is to make it easier to work with finite element models and BIM models and act as a high-level scripting library for parametric and procedural modelling techniques.

### Paradoc

paradoc was created to simplify the generation of reports by creating the structure of the document and text in markdown with a string substitution scheme that lets you easily pass in tables, functions and finally be able to produce production ready documents in Microsoft Word.

# Summary

A summary of all performed analyses will be performed in this section and a percentage difference for each FEA solver per type of analysis will be calculated and presented.

# Analyses

## Eigenvalue Analysis

A simple cantilevered beam is subjected to eigenvalue analysis with a total of 11 eigenmodes requested. 47 cases are compared across solvers: abaqus, calculix, code_aster, sesam.

The beam model will be analyzed by varying the following parameters:

- Mesh type (Solid, Shell, Line)

- Mesh element types (TET, HEX, TRI, QUAD)

- Mesh element order (1st and 2nd order)

- Reduced integration elements (R)

- Different FEA solvers (Calculix, Code Aster, Abaqus, Sesam)

### Model Description

| Property | Value |
| --- | --- |
| Description | Beam(“MyBeam”, [0.0, 0.5, 0.5], [3.0, 0.5, 0.5], “IPE400”, “S420”) |
| Length | 3.00 m |
| Section | IPE400 |
| Material | S420 |
| Young’s modulus | 2.100e+11 Pa |
| Yield stress | 4.200e+08 Pa |
| Density | 7850 kg/m³ |

Figure 1: Cantilever beam geometry.

### Summary

This section presents a comparison between the calculated results from the eigenvalue analysis of a cantilever beam using the different FEA tools.

Note! The Sestra and Abaqus analyses are not performed as part of the github actions step. The eigenmode results are kept in cached json files and imported during the compilation of the report using paradoc.

#### Eigenfrequency vs. mode number

Figure 2: Eigenfrequency vs. mode number across FEA cases.

#### Comparison tables

Columns are sortable; the table grammar {tbl:sortby:Mode:asc;index:no} sets the initial sort and hides the index column.

Table 1-1: Eigenfrequency comparison (Hz) — solid, 1st order.

| Mode | ses_HEX | ses_TET | aba_HEXR | aba_HEX | aba_TET | ccx_TET | ca_TET | ccx_HEX | ca_HEX | ccx_HEXR | ca_HEXR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12.9766 | 15.7932 | 12.514 | 13.435 | 15.79 | 15.9285 | 15.9277 | 13.6205 | 13.6205 | 14.3018 | 13.5291 |
| 2 | 20.3015 | 51.5418 | 34.157 | 20.731 | 51.532 | 51.5896 | 51.5872 | 21.0685 | 21.0685 | 51.6653 | 34.7401 |
| 3 | 50.6181 | 97.7314 | 49.598 | 50.678 | 97.588 | 102.446 | 102.41 | 50.8252 | 50.8252 | 88.2331 | 50.7365 |
| 4 | 77.3921 | 105.691 | 55.271 | 82.799 | 104.8 | 116.744 | 116.512 | 84.066 | 84.066 | 116.873 | 82.2962 |
| 5 | 98.1185 | 260.887 | 64.305 | 100.19 | 260.59 | 261.877 | 261.806 | 102.334 | 102.334 | 240.766 | 123.303 |
| 6 | 133.016 | 267.131 | 74.866 | 223.14 | 266.26 | 284.697 | 284.465 | 227.25 | 227.25 | 258.286 | 147.139 |
| 7 | 153.833 | 287.461 | 90.033 | 254.98 | 285.05 | 357.062 | 356.306 | 255.872 | 255.872 | 351.823 | 205.267 |
| 8 | 178.565 | 432.347 | 124.58 | 260.1 | 432.31 | 432.36 | 432.35 | 266.519 | 266.519 | 431.955 | 243.985 |
| 9 | 214.689 | 503.862 | 155.99 | 396.57 | 500.86 | 529.297 | 528.489 | 407.869 | 407.869 | 439.038 | 254.838 |
| 10 | 253.463 | 539.506 | 159.95 | 431.8 | 534.26 | 601.943 | 601.586 | 432.207 | 432.207 | 456.762 | 282.888 |
| 11 | 256.05 | 598.2 | 162.56 | 486.96 | 596.78 | 617.646 | 616.155 | 500.931 | 500.931 | 582.562 | 305.247 |

Table 1-1: Eigenfrequency comparison (Hz) — solid, 2nd order.

| Mode | ses_HEX | ses_TET | aba_HEXR | aba_HEX | aba_TET | ccx_TET | ca_TET | ccx_HEX | ca_HEX | ccx_HEXR | ca_HEXR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12.9541 | 12.9537 | 12.943 | 12.955 | 12.954 | 12.9542 | 12.9542 | 12.9551 | 12.9551 | 12.9428 | 12.9427 |
| 2 | 20.0445 | 20.2945 | 20.029 | 20.15 | 20.294 | 20.2876 | 20.2877 | 20.1503 | 20.1503 | 20.0288 | 20.0288 |
| 3 | 50.5342 | 50.5401 | 50.498 | 50.541 | 50.54 | 50.5401 | 50.5402 | 50.5412 | 50.5412 | 50.4977 | 50.4977 |
| 4 | 76.7467 | 77.1278 | 76.687 | 76.89 | 77.128 | 77.2297 | 77.2297 | 76.8903 | 76.8903 | 76.6876 | 76.6875 |
| 5 | 97.4723 | 97.8296 | 97.391 | 97.594 | 97.83 | 97.8506 | 97.8505 | 97.5939 | 97.5939 | 97.3913 | 97.3912 |
| 6 | 130.865 | 132.478 | 130.81 | 131.37 | 132.48 | 133.015 | 133.014 | 131.365 | 131.365 | 130.814 | 130.809 |
| 7 | 148.496 | 153.631 | 148.47 | 149.96 | 153.63 | 154.804 | 154.802 | 149.958 | 149.958 | 148.478 | 148.474 |
| 8 | 172.243 | 178.114 | 172.17 | 173.92 | 178.12 | 181.029 | 181.026 | 173.925 | 173.925 | 172.177 | 172.171 |
| 9 | 201.904 | 215.815 | 201.84 | 205.29 | 215.82 | 219.501 | 219.497 | 205.292 | 205.292 | 201.846 | 201.839 |
| 10 | 246.353 | 253.203 | 246.17 | 251.35 | 253.2 | 253.193 | 253.193 | 251.347 | 251.347 | 246.177 | 246.165 |
| 11 | 252.742 | 254.466 | 252.57 | 252.92 | 254.47 | 254.685 | 254.684 | 252.917 | 252.917 | 252.573 | 252.573 |

Table 1-1: Eigenfrequency comparison (Hz) — shell, 1st order.

| Mode | ses_QUAD | ses_TRI | aba_QUADR | aba_QUAD | aba_TRIR | ccx_TRI | ca_TRI | ccx_QUAD | ca_QUAD | ccx_QUADR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12.7628 | 13.6414 | 12.373 | 12.78 | 13.6869 | 13.7254 | 13.642 | 12.7639 | 13.1463 | 14.2558 |
| 2 | 19.3943 | 20.1983 | 19.23 | 19.593 | 20.3643 | 36.6896 | 20.4181 | 19.6304 | 19.9719 | 53.2618 |
| 3 | 51.9912 | 52.1922 | 51.849 | 51.998 | 52.1874 | 52.2485 | 52.1927 | 51.9997 | 52.0595 | 87.9153 |
| 4 | 75.1918 | 79.5226 | 73.558 | 75.683 | 80.2058 | 83.7093 | 79.6596 | 75.4574 | 77.2361 | 121.85 |
| 5 | 95.9297 | 101.756 | 93.709 | 96.393 | 102.043 | 132.928 | 102.01 | 96.2959 | 98.7619 | 239.804 |
| 6 | 125.116 | 125.093 | 126.87 | 126.96 | 124.093 | 218.167 | 125.459 | 125.898 | 125.442 | 262.83 |
| 7 | 141.556 | 143.164 | 144.58 | 145.57 | 145.486 | 259.245 | 144.895 | 145.282 | 143.864 | 365.1 |
| 8 | 165.03 | 166.668 | 168.38 | 169.39 | 168.62 | 297.856 | 168.951 | 169.441 | 167.974 | 431.774 |
| 9 | 190.099 | 191.533 | 197.51 | 198.75 | 199.719 | 304.464 | 195.184 | 200.494 | 193.96 | 451.745 |
| 10 | 229.46 | 230.489 | 241.27 | 242.55 | 242.995 | 365.943 | 235.115 | 247.276 | 233.817 | 464.44 |
| 11 | 250.786 | 257.362 | 245.07 | 252.49 | 257.447 | 368.764 | 257.409 | 251.554 | 257.105 | 589.24 |

Table 1-1: Eigenfrequency comparison (Hz) — shell, 2nd order.

| Mode | ses_QUAD | ses_TRI | aba_QUADR | aba_QUAD | aba_TRI | ccx_TRI | ca_TRI | ccx_QUAD | ca_QUAD | ccx_QUADR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 12.7554 | 12.7558 | 12.758 | 12.758 | 12.756 | 12.7562 | 12.7566 | 12.7576 | 12.7561 | 12.7559 |
| 2 | 19.4731 | 19.5048 | 19.486 | 19.486 | 19.567 | 19.5254 | 19.4509 | 19.5638 | 19.4736 | 19.5021 |
| 3 | 51.9702 | 51.9704 | 51.998 | 51.998 | 51.969 | 51.974 | 51.97 | 51.9801 | 51.97 | 51.9741 |
| 4 | 75.1245 | 75.1716 | 75.149 | 75.149 | 75.145 | 75.1848 | 75.1443 | 75.2685 | 75.1252 | 75.1399 |
| 5 | 95.8886 | 95.9337 | 95.92 | 95.92 | 96.001 | 95.9713 | 95.8707 | 96.0372 | 95.8893 | 95.9349 |
| 6 | 124.987 | 125.173 | 125 | 125 | 125.21 | 125.465 | 125.087 | 125.889 | 124.987 | 125.257 |
| 7 | 142.234 | 142.768 | 142.32 | 142.32 | 142.77 | 142.995 | 142.359 | 143.72 | 142.233 | 142.497 |
| 8 | 165.718 | 166.334 | 165.85 | 165.85 | 166.6 | 166.579 | 165.79 | 167.347 | 165.719 | 165.981 |
| 9 | 191.999 | 193.308 | 192.18 | 192.18 | 192.84 | 194.014 | 192.406 | 195.577 | 191.998 | 192.87 |
| 10 | 232.626 | 234.613 | 232.87 | 232.87 | 233.36 | 236.042 | 233.389 | 238.389 | 232.628 | 234.347 |
| 11 | 250.095 | 250.24 | 250.17 | 250.17 | 250.13 | 250.287 | 250.175 | 250.521 | 250.096 | 250.147 |

Table 1-1: Eigenfrequency comparison (Hz) — line, 1st order.

| Mode | ses | aba | ca |
| --- | --- | --- | --- |
| 1 | 12.2454 | 10.785 | 12.2454 |
| 2 | 12.9431 | 12.928 | 12.9793 |
| 3 | 36.7525 | 32.342 | 36.7525 |
| 4 | 44.9622 | 50.543 | 52.9559 |
| 5 | 61.3087 | 53.855 | 61.3087 |
| 6 | 79.793 | 75.296 | 81.3401 |
| 7 | 85.9466 | 79.59 | 85.9466 |
| 8 | 110.699 | 96.637 | 110.699 |
| 9 | 135.6 | 117.85 | 135.6 |
| 10 | 160.681 | 138.9 | 160.681 |
| 11 | 176.495 | 159.77 | 185.976 |

Table 1-1: Eigenfrequency comparison (Hz) — line, 2nd order.

| Mode | ses | aba |
| --- | --- | --- |
| 1 | 10.039 | 10.786 |
| 2 | 10.6144 | 12.932 |
| 3 | 30.1078 | 32.358 |
| 4 | 36.752 | 50.557 |
| 5 | 50.1488 | 53.93 |
| 6 | 65.3265 | 75.502 |
| 7 | 70.1429 | 79.684 |
| 8 | 90.0695 | 97.074 |
| 9 | 109.907 | 118.65 |
| 10 | 129.631 | 140.22 |
| 11 | 142.572 | 161.8 |

#### Effective modal mass

Effective modal mass [kg] per case, summed over the captured modes in the global X / Y / Z directions. Every solver reports it; Code Aster and Sesam give translational mass only. Summing over enough modes approaches the structure’s total mass in each direction.

Table 1-1: Effective modal mass [kg] per case, summed over the captured modes in the global X/Y/Z directions. Code_Aster and Sesam report translational effective mass only.

| Case | Modes | ΣMeff X | ΣMeff Y | ΣMeff Z |
| --- | --- | --- | --- | --- |
| aba_line_o1 | 11 | 0 | 152.8 | 118.2 |
| aba_line_o2 | 11 | 0 | 152.8 | 118.2 |
| aba_shell_o1_QUAD | 11 | 0 | 168 | 122 |
| aba_shell_o1_QUADR | 11 | 0 | 168.4 | 122 |
| aba_shell_o2_QUAD | 11 | 0 | 167.8 | 122 |
| aba_shell_o2_QUADR | 11 | 0 | 167.8 | 122 |
| aba_shell_o2_TRI | 11 | 0 | 167.9 | 122 |
| aba_solid_o1_HEX | 11 | 153.5 | 170.5 | 158.2 |
| aba_solid_o1_HEXR | 11 | 0 | 154.3 | 119 |
| aba_solid_o1_TET | 11 | 153.4 | 172 | 170.9 |
| aba_solid_o2_HEX | 11 | 0 | 163.3 | 158.6 |
| aba_solid_o2_HEXR | 11 | 0 | 163.2 | 158.7 |
| aba_solid_o2_TET | 11 | 0 | 162 | 158.6 |
| ca_line_o1 | 11 | 0 | 152.3 | 116.5 |
| ca_shell_o1_QUAD | 11 | 0 | 167.6 | 163.7 |
| ca_shell_o1_TRI | 11 | 0 | 167.3 | 163.7 |
| ca_shell_o2_QUAD | 11 | 0 | 167.8 | 122 |
| ca_shell_o2_TRI | 11 | 0 | 167.8 | 122 |
| ca_solid_o1_HEX | 11 | 153.5 | 170.8 | 158.3 |
| ca_solid_o1_HEXR | 11 | 0 | 164.7 | 158.5 |
| ca_solid_o1_TET | 11 | 153.4 | 172.6 | 171 |
| ca_solid_o2_HEX | 11 | 0 | 163.3 | 158.6 |
| ca_solid_o2_HEXR | 11 | 0 | 163.2 | 158.7 |
| ca_solid_o2_TET | 11 | 0 | 162.2 | 158.6 |
| ccx_shell_o1_QUAD | 11 | 0 | 168.1 | 122 |
| ccx_shell_o1_QUADR | 11 | 158 | 176.8 | 176.5 |
| ccx_shell_o1_TRI | 11 | 0 | 174.8 | 163.6 |
| ccx_shell_o2_QUAD | 11 | 0 | 167.8 | 122 |
| ccx_shell_o2_QUADR | 11 | 0 | 167.8 | 122 |
| ccx_shell_o2_TRI | 11 | 0 | 167.8 | 122 |
| ccx_solid_o1_HEX | 11 | 153.4 | 170.7 | 158.2 |
| ccx_solid_o1_HEXR | 11 | 153.5 | 171.8 | 171.3 |
| ccx_solid_o1_TET | 11 | 153.4 | 172.6 | 171 |
| ccx_solid_o2_HEX | 11 | 0 | 163.3 | 158.6 |
| ccx_solid_o2_HEXR | 11 | 0 | 163.2 | 158.7 |
| ccx_solid_o2_TET | 11 | 0 | 162.2 | 158.6 |
| ses_line_o1 | 11 | 0 | 153 | 166.4 |
| ses_line_o2 | 11 | 0 | 231.8 | 252.3 |
| ses_shell_o1_QUAD | 11 | 0 | 167.6 | 122 |
| ses_shell_o1_TRI | 11 | 0 | 167.1 | 163.6 |
| ses_shell_o2_QUAD | 11 | 0 | 167.8 | 122 |
| ses_shell_o2_TRI | 11 | 0 | 167.9 | 122 |
| ses_solid_o1_HEX | 11 | 0 | 161.7 | 158.6 |
| ses_solid_o1_TET | 11 | 153.4 | 172.1 | 170.9 |
| ses_solid_o2_HEX | 11 | 0 | 163.1 | 158.7 |
| ses_solid_o2_TET | 11 | 0 | 162 | 158.6 |

The per-mode effective masses and participation factors, compared across solvers for every mesh configuration, are in the appendix.

Short description:

Fem formats

- ccx: Calculix

- ca: Code Aster

- ses: Sesam

- aba: Abaqus

Element Types

- TET: Tetrahedrons

- HEX: Hexahedrons

- TRI: Triangle

- QUAD: Quadrilateral

Reduced Integration

All element types ending with capital R (QUADR, HEXR etc.) are reduced integration elements.

## Nonlinear Analysis

A simple cantilever beam with a series of cutouts is subjugated to a large vertical acceleration field. The level of plastic strains are monitored for comparison across different FE tools.

### Model Description

### Summary

## Plate Analysis

The analyses above are one beam, in three representations. This section is the plate case: an ada.Plate object, meshed into shell elements by adapy’s own meshing module, written out as an input deck per solver, and checked against a closed form as well as against the other solvers.

Nothing here drives a solver through its API. Each solver reads a file adapy wrote — an .inp, a .fem, a .comm/.med — which is the same route a user’s own model takes, so what this section verifies is the thing adapy actually ships.

9 cases ran on the machine that built this report, using: calculix, code_aster. The remaining columns come from cached results measured elsewhere, the same arrangement the eigenvalue section uses for Abaqus and Sestra.

### Model Description

A rectangular strip, simply supported on its two short edges and held in cylindrical bending on the two long ones, under a uniform surface pressure.

| Property | Value |
| --- | --- |
| Span | 4.00 m |
| Width | 0.50 m |
| Thickness | 0.010 m |
| Pressure | 1000 Pa |
| Material | S355 (E = 210 GPa, ν = 0.3, ρ = 7850 kg/m³) |

Figure 1: Plate strip geometry.

Figure 2: Plate strip with a T-profile stiffener on its centreline.

The span-to-thickness ratio is 400, so transverse shear sits several orders below the discretisation error and the thin-plate closed forms are the right reference rather than an approximation of one.

The supports are written as four node sets rather than one per edge. Simple support and cylindrical bending overlap at the four corners, and the obvious spelling constrains u2 twice there: Abaqus and Sestra accept that, while Code_Aster refuses it outright (<ASSEMBLA_26> … est bloqué plusieurs fois). A model that only some solvers will read is not a cross-solver case, so the corner nodes carry only the rotation the long edges add.

### Static analysis under pressure

The static case is the only one that reaches the distributed-load records — Abaqus’ *Dsload, Calculix’ *DLOAD, Code_Aster’s FORCE_COQUE and Sesam’s BEUSLO. The eigenvalue analyses reach the mass and stiffness matrices and nothing else, so a pressure written wrongly is invisible to them.

Mid-span deflection against the closed form 5 q L⁴ / (384 D), with D = E t³ / (12 (1 − ν²)) — which evaluates to 0.173333 m for this strip. Halving the seed should quarter the remaining error; the three rows are a factor of two apart so the order is readable off the table rather than asserted.

Table 1-1: Plate strip mid-span deflection [m] under 1 kPa, by solver and mesh seed. The closed form is 5 q L^4 / (384 D) for a strip in cylindrical bending.

| Seed [m] | aba | ca | ccx | ses | closed form |
| --- | --- | --- | --- | --- | --- |
| 0.125 | 0.173065 | 0.173198 | 0.173065 | 0.173198 | 0.173333 |
| 0.0625 | 0.173269 | 0.173299 | 0.173268 | 0.173299 | 0.173333 |
| 0.03125 | 0.173319 | 0.173325 | 0.173319 | 0.173325 | 0.173333 |

The same strip with a T-profile stiffener along its centreline, meshed as line elements sharing the shells’ nodes. There is no closed form for it — a parallel-spring estimate is not one — so this is an agreement case between solvers, and it is what carries a T section through the deck writers.

Table 1-1: The same strip with a T-profile stiffener on its centreline [m]. No closed form: this is a cross-solver agreement case.

| Seed [m] | aba_st | ca_st | ses_st |
| --- | --- | --- | --- |
| 0.125 | 0.000584531 | 0.000710295 | 0.00073971 |
| 0.0625 | 0.000585478 | 0.000710711 | 0.000738964 |
| 0.03125 | 0.000585996 | 0.000710815 | 0.000736999 |

#### Sign convention

Every writer above places a positive pressure so that it pushes into the face it names. That is Abaqus’ convention, and it has to be shared, because a load direction that depends on the output format makes a cross-solver comparison meaningless.

It is worth stating which face that is, because it is not a global direction: gmsh winds every element of this strip with its normal along −z, so the positive face looks down and a positive pressure on it moves the strip up. The deflections above are therefore positive. tests/fem/test_pressure_load_cross_format.py pins this per format.

### Eigenvalue analysis

The same plate, as an eigenvalue analysis, against f_n = n² π / (2 L²) √(D / (ρ t)) — the simply supported beam frequencies with the beam’s EI / (ρ A) replaced by the strip’s D / (ρ t). Valid for the cylindrical modes, which are the ones the long-edge constraint admits.

Table 1-1: Plate strip eigenfrequencies [Hz], by solver, at a 0.0625 m seed. The closed form is f_n = n^2 pi / (2 L^2) sqrt(D / (rho t)), valid for the cylindrical modes the long-edge constraint admits.

| Mode | aba | ca | ccx | ses | closed form |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.5371 | 1.53661 | 1.53675 | 1.53661 | 1.53661 |
| 2 | 6.1536 | 6.14644 | 6.14861 | 6.14644 | 6.14644 |
| 3 | 13.866 | 13.8295 | 13.8405 | 13.8295 | 13.8295 |
| 4 | 24.7 | 24.5858 | 24.6205 | 24.5858 | 24.5857 |
| 5 | 38.695 | 38.4153 | 38.5002 | 38.4153 | 38.4152 |
| 6 | 55.9 | 55.3181 | 55.4943 | 55.3182 | 55.3179 |

Short description of the solver column names:

- ccx: Calculix

- ca: Code Aster

- ses: Sesam

- aba: Abaqus

A _st suffix marks the stiffened variant.

- Detailed results

Mode shapes and deflected shapes from every format that ran on the machine that built this report, laid out so the formats can be read against each other: one section per mesh configuration, one heading per mode, and the formats side by side under it. A swapped mode pair or a differently deflected shape shows up as two pictures that do not match.

Abaqus and Sesam cannot run where this report is built. Their mode shapes are the raw results a licensed machine committed beside its cached frequencies, and the figures are rendered from them here; a case with no committed results has no figures. Solver versions: Abaqus v2025, Calculix v2.23, Code Aster v18.1.7, Sesam v11.3.1.0.

- Cantilever modal participation

Per-mode effective modal mass and participation factors in the global X / Y / Z directions, for every mesh configuration, with the formats side by side. Code Aster and Sesam report the translational directions only.

A participation factor Γ is relative to how the solver normalised its eigenvectors: Abaqus scales the largest displacement to 1, the others scale to unit generalised mass. Raw factors from different solvers therefore do not compare. The tables give the factor scaled to unit generalised mass, sign(Γ)·√Meff, which is what the others report as-is. Its sign is the solver’s choice of eigenvector sign and may differ from mode to mode. The effective mass Meff = Γ²·m_gen does not depend on the normalisation.

- Solid, 1st order

Table 1-1: Effective modal mass [kg] per mode, global X/Y/Z — solid, order 1.

| Mode | Direction | ses_HEX | ses_TET | aba_HEXR | aba_HEX | aba_TET | ccx_TET | ca_TET | ccx_HEX | ca_HEX | ccx_HEXR | ca_HEXR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | Y | 116.4 | 117.5 | 116.9 | 116.4 | 117.5 | 118.2 | 118.2 | 116.4 | 116.4 | 116.5 | 116.4 |
| 1 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Z | 0 | 118.3 | 0 | 0 | 118.2 | 118.4 | 118.4 | 0 | 0 | 118.5 | 0 |
| 3 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | Y | 0 | 34.9 | 0 | 0 | 34.9 | 35.5 | 35.5 | 0 | 0 | 36.2 | 0 |
| 3 | Z | 118.3 | 0 | 119 | 118.2 | 0 | 0 | 0 | 118.2 | 118.2 | 0 | 118.3 |
| 4 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | Y | 35.9 | 0 | 2.1 | 36 | 0 | 0 | 0 | 36 | 36.1 | 0 | 36.1 |
| 4 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12.6 | 0 |
| 5 | Z | 0 | 39.8 | 0 | 0 | 39.7 | 39.9 | 39.9 | 0 | 0 | 0 | 0 |
| 6 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | Y | 0 | 13.3 | 31.4 | 12.4 | 13.2 | 12.8 | 12.8 | 12.5 | 12.5 | 0 | 0 |
| 6 | Z | 0 | 0 | 0 | 0 | 0.1 | 0 | 0 | 0 | 0 | 40.2 | 0 |
| 7 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | Y | 3.2 | 0 | 2.8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 8.9 |
| 7 | Z | 0 | 0 | 0 | 40 | 0 | 0 | 0 | 40 | 40.1 | 0 | 0 |
| 8 | X | 0 | 153.4 | 0 | 0 | 153.4 | 153.4 | 153.4 | 0 | 0 | 153.5 | 0 |
| 8 | Y | 5.7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3.1 |
| 8 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.3 |
| 9 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | Y | 0.4 | 6.3 | 1.1 | 5.7 | 6.3 | 6.1 | 6.1 | 5.9 | 5.9 | 0.4 | 0 |
| 9 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 39.9 |
| 10 | X | 0 | 0 | 0 | 153.5 | 0 | 0 | 0 | 153.4 | 153.5 | 0 | 0 |
| 10 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6.1 | 0 |
| 10 | Z | 40.3 | 0 | 0 | 0 | 0 | 12.8 | 12.8 | 0 | 0 | 0 | 0 |
| 11 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | Z | 0 | 12.8 | 0 | 0 | 12.8 | 0 | 0 | 0 | 0 | 12.7 | 0 |

Table 1-1: Participation factor per mode scaled to unit generalised mass [√kg], global X/Y/Z — solid, order 1.

| Mode | Direction | ses_HEX | ses_TET | aba_HEXR | aba_HEX | aba_TET | ccx_TET | ca_TET | ccx_HEX | ca_HEX | ccx_HEXR | ca_HEXR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | -0 | 0 | 0 | -0 | 0 | -0 | 0 | -0 | 0 | 0 | 0 |
| 1 | Y | 10.79 | -10.841 | 10.813 | 10.788 | 10.838 | -10.871 | 10.871 | 10.787 | 10.787 | 10.794 | 10.791 |
| 1 | Z | 0 | 0.017 | -0 | -0 | -0.017 | 0.019 | -0.019 | -0 | -0 | 0 | -0.001 |
| 2 | X | -0 | -0 | 0 | 0 | 0 | 0 | 0 | -0 | -0 | -0 | 0.001 |
| 2 | Y | -0 | -0.028 | 0 | 0 | 0.028 | 0.03 | 0.03 | 0 | 0 | 0 | 0.022 |
| 2 | Z | 0 | -10.876 | -0 | -0 | 10.874 | 10.88 | 10.88 | -0 | 0 | -10.884 | -0 |
| 3 | X | -0 | 0 | -0 | -0 | -0 | 0 | -0 | -0 | -0 | 0 | 0 |
| 3 | Y | -0 | 5.911 | 0 | 0 | -5.909 | 5.961 | -5.961 | 0 | 0 | -6.013 | 0 |
| 3 | Z | 10.876 | -0.019 | 10.907 | 10.872 | 0.019 | -0.02 | 0.02 | 10.871 | 10.872 | -0 | 10.876 |
| 4 | X | -0 | -0.003 | -0 | -0 | 0.003 | 0.003 | 0.003 | 0 | 0 | 0 | -0 |
| 4 | Y | 5.995 | 0.069 | -1.456 | -6.002 | -0.075 | -0.108 | -0.11 | 6.004 | -6.004 | -0 | -6.01 |
| 4 | Z | -0 | -0.003 | 0 | 0 | 0.003 | 0.007 | 0.008 | 0 | -0 | -0 | -0.004 |
| 5 | X | 0 | -0.001 | -0 | 0 | -0.001 | 0 | -0 | -0 | 0 | -0 | -0.002 |
| 5 | Y | -0 | 0.122 | -0 | -0 | 0.132 | -0.028 | 0.029 | 0 | -0 | -3.549 | -0.084 |
| 5 | Z | -0 | -6.305 | 0 | 0 | -6.305 | 6.313 | -6.315 | -0 | -0 | 0 | 0.006 |
| 6 | X | -0 | 0 | 0 | -0 | 0 | 0 | -0.001 | 0 | -0 | 0 | -0.001 |
| 6 | Y | -0.114 | 3.642 | -5.603 | 3.522 | 3.638 | -3.576 | 3.577 | 3.53 | 3.532 | -0 | 0.192 |
| 6 | Z | 0 | 0.207 | -0 | 0 | 0.226 | -0.056 | 0.057 | 0 | -0 | -6.34 | -0.015 |
| 7 | X | -0 | 0 | -0 | -0 | 0 | -0.021 | -0.021 | 0 | 0 | 0 | 0 |
| 7 | Y | -1.788 | 0.027 | -1.671 | 0 | 0.03 | 0.04 | 0.04 | -0 | -0 | -0 | 2.99 |
| 7 | Z | 0 | -0.033 | 0 | -6.326 | -0.035 | -0.02 | -0.02 | 6.327 | -6.329 | -0 | 0.071 |
| 8 | X | -0 | 12.384 | -0 | -0 | 12.385 | 12.384 | 12.386 | 0 | -0 | 12.389 | 0.001 |
| 8 | Y | 2.39 | -0.001 | -0 | 0 | -0.001 | 0 | 0 | 0 | 0 | 0 | 1.76 |
| 8 | Z | 0 | 0 | -0 | 0 | 0 | -0 | -0 | -0 | -0 | 0 | -0.503 |
| 9 | X | -0 | -0.005 | -0 | 0 | -0.006 | 0.001 | 0.001 | -0 | -0 | 0 | 0.001 |
| 9 | Y | 0.662 | -2.518 | -1.056 | -2.385 | -2.515 | -2.462 | -2.464 | 2.426 | -2.43 | -0.619 | -0.115 |
| 9 | Z | -0 | 0.007 | -0 | 0 | 0.007 | 0.064 | 0.064 | 0 | 0 | -0 | -6.318 |
| 10 | X | -0 | -0.015 | 0.019 | 12.391 | -0.016 | 0.001 | -0.001 | -12.385 | 12.388 | -0 | 0.006 |
| 10 | Y | 0 | -0.015 | -0 | 0 | -0.017 | -0.045 | 0.045 | -0 | -0 | 2.478 | 0.086 |
| 10 | Z | -6.346 | 0.003 | 0 | -0 | 0.003 | -3.572 | 3.576 | 0 | 0 | -0 | -0.006 |
| 11 | X | 0 | 0.001 | -0 | 0 | -0.001 | -0.012 | 0.012 | 0 | 0 | -0 | 0.005 |
| 11 | Y | -0 | 0 | 0 | 0 | -0 | 0.016 | -0.016 | -0 | 0 | -0 | 0.009 |
| 11 | Z | -0 | -3.578 | -0 | 0 | 3.578 | 0.029 | -0.031 | 0 | -0 | 3.558 | -0.001 |

Solid, 2nd order

Table 1-1: Effective modal mass [kg] per mode, global X/Y/Z — solid, order 2.

| Mode | Direction | ses_HEX | ses_TET | aba_HEXR | aba_HEX | aba_TET | ccx_TET | ca_TET | ccx_HEX | ca_HEX | ccx_HEXR | ca_HEXR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | Y | 116.5 | 116.5 | 116.6 | 116.5 | 116.5 | 116.5 | 116.5 | 116.5 | 116.5 | 116.6 | 116.6 |
| 1 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | Z | 118.4 | 118.4 | 118.4 | 118.4 | 118.4 | 118.4 | 118.4 | 118.4 | 118.4 | 118.4 | 118.4 |
| 4 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | Y | 35.9 | 36 | 35.9 | 35.9 | 36 | 36 | 36 | 35.9 | 35.9 | 35.9 | 35.9 |
| 4 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | Y | 3.4 | 3.1 | 3.4 | 3.3 | 3.1 | 3.2 | 3.2 | 3.3 | 3.3 | 3.4 | 3.4 |
| 7 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | Y | 5.3 | 5.8 | 5.3 | 5.4 | 5.8 | 5.9 | 5.9 | 5.4 | 5.4 | 5.3 | 5.3 |
| 8 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | Y | 0.6 | 0.6 | 0.6 | 0.5 | 0.6 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.6 |
| 9 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | Y | 1.5 | 0 | 1.5 | 1.6 | 0 | 0 | 0 | 1.6 | 1.6 | 1.5 | 1.5 |
| 10 | Z | 0 | 40.2 | 0 | 0 | 40.2 | 40.2 | 40.2 | 0 | 0 | 0 | 0 |
| 11 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | Z | 40.3 | 0 | 40.3 | 40.3 | 0 | 0 | 0 | 40.3 | 40.3 | 40.3 | 40.3 |

Table 1-1: Participation factor per mode scaled to unit generalised mass [√kg], global X/Y/Z — solid, order 2.

| Mode | Direction | ses_HEX | ses_TET | aba_HEXR | aba_HEX | aba_TET | ccx_TET | ca_TET | ccx_HEX | ca_HEX | ccx_HEXR | ca_HEXR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | 0 | -0 | -0 | 0 | 0 | -0 | 0 | -0 | -0 | 0 | 0 |
| 1 | Y | 10.794 | -10.794 | 10.796 | 10.794 | 10.794 | -10.794 | 10.794 | 10.794 | 10.794 | 10.796 | 10.796 |
| 1 | Z | 0 | -0 | 0 | 0 | 0 | -0 | 0 | -0 | 0 | 0 | -0 |
| 2 | X | 0 | 0 | -0 | -0 | 0 | -0 | 0 | -0 | -0 | -0 | 0 |
| 2 | Y | -0 | -0 | 0 | 0 | -0 | -0 | 0 | 0 | 0 | -0 | 0 |
| 2 | Z | 0 | 0 | -0 | 0 | 0 | -0 | 0 | 0 | 0 | 0 | -0 |
| 3 | X | 0 | 0 | -0 | -0 | 0 | -0 | -0 | -0 | -0 | -0 | -0 |
| 3 | Y | 0 | -0 | -0 | -0 | -0 | -0 | -0 | 0 | -0 | -0 | 0 |
| 3 | Z | 10.88 | 10.879 | 10.882 | 10.879 | 10.879 | 10.879 | 10.879 | 10.879 | 10.879 | 10.882 | 10.882 |
| 4 | X | -0 | 0 | -0 | -0 | 0 | -0 | -0 | 0 | 0 | 0 | -0 |
| 4 | Y | -5.991 | -5.997 | -5.992 | -5.993 | -5.997 | -6.004 | -6.004 | -5.993 | -5.993 | -5.992 | -5.992 |
| 4 | Z | 0 | -0 | -0 | -0 | -0 | 0 | 0 | 0 | -0 | -0 | -0 |
| 5 | X | -0 | 0 | 0 | 0 | -0 | -0 | 0 | 0 | -0 | -0 | -0 |
| 5 | Y | 0 | 0.001 | -0 | -0 | -0.001 | 0.001 | -0.001 | 0 | -0 | -0 | -0 |
| 5 | Z | -0 | -0 | -0 | -0 | 0 | -0 | 0 | -0 | 0 | 0 | 0 |
| 6 | X | 0 | -0 | -0 | 0 | 0 | 0 | 0 | 0 | -0 | -0 | 0 |
| 6 | Y | -0.096 | -0.119 | -0.097 | -0.103 | 0.119 | -0.093 | -0.093 | 0.103 | -0.103 | 0.096 | -0.097 |
| 6 | Z | 0 | 0 | 0 | -0 | -0 | 0 | 0 | 0 | -0 | 0 | -0 |
| 7 | X | 0 | 0 | -0 | 0 | -0 | 0 | 0 | -0 | 0 | 0 | 0 |
| 7 | Y | -1.839 | -1.768 | 1.84 | 1.818 | 1.768 | 1.799 | 1.799 | 1.818 | 1.818 | 1.84 | 1.84 |
| 7 | Z | -0 | 0 | -0 | 0 | -0 | 0 | 0 | 0 | 0 | 0 | -0 |
| 8 | X | 0 | 0 | 0 | 0 | -0 | 0 | 0 | -0 | -0 | 0 | 0 |
| 8 | Y | -2.305 | -2.406 | 2.304 | 2.328 | 2.405 | 2.436 | 2.435 | 2.328 | 2.328 | 2.304 | 2.304 |
| 8 | Z | -0 | 0 | 0 | -0 | -0 | -0 | -0 | -0 | -0 | 0 | -0 |
| 9 | X | -0 | 0 | -0 | 0 | -0 | 0 | 0 | -0 | 0 | -0 | -0 |
| 9 | Y | -0.746 | -0.774 | -0.747 | -0.719 | 0.774 | 0.69 | 0.69 | 0.72 | -0.719 | 0.747 | -0.747 |
| 9 | Z | 0 | -0 | 0 | 0 | 0 | -0 | -0 | 0 | -0 | 0 | -0 |
| 10 | X | -0 | -0 | 0 | 0 | -0 | -0 | 0 | -0 | -0 | 0 | 0 |
| 10 | Y | 1.211 | 0 | 1.212 | 1.267 | 0 | -0 | 0 | -1.267 | 1.267 | -1.213 | 1.212 |
| 10 | Z | 0 | -6.344 | -0 | -0 | -6.344 | 6.344 | -6.344 | 0 | -0 | 0 | -0 |
| 11 | X | -0 | -0 | 0 | 0 | -0 | -0 | 0 | -0 | -0 | 0 | -0 |
| 11 | Y | 0 | -0.001 | 0 | 0 | -0.001 | -0.008 | 0.008 | 0 | -0 | -0 | -0 |
| 11 | Z | 6.346 | -0.006 | -6.345 | -6.344 | -0.006 | 0.004 | -0.004 | 6.345 | -6.344 | 6.345 | -6.345 |

Shell, 1st order

Table 1-1: Effective modal mass [kg] per mode, global X/Y/Z — shell, order 1.

| Mode | Direction | ses_QUAD | ses_TRI | aba_QUADR | aba_QUAD | ccx_TRI | ca_TRI | ccx_QUAD | ca_QUAD | ccx_QUADR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | Y | 119.9 | 119.9 | 120 | 119.9 | 119.7 | 119.9 | 119.9 | 119.9 | 119.9 |
| 1 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 122.1 |
| 3 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 37.2 |
| 3 | Z | 122 | 122 | 122 | 122 | 122.1 | 122 | 122 | 122 | 0 |
| 4 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | Y | 36.8 | 36.6 | 37 | 36.9 | 37.3 | 36.6 | 36.9 | 36.8 | 0 |
| 4 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 13 |
| 5 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | Y | 0 | 0.1 | 0 | 0 | 12.7 | 0.1 | 0 | 0 | 0 |
| 6 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 41.5 |
| 7 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | Y | 3.5 | 2.7 | 3.8 | 3.3 | 0 | 2.7 | 3.3 | 3.2 | 0 |
| 7 | Z | 0 | 0 | 0 | 0 | 41.6 | 0 | 0 | 0 | 0 |
| 8 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 158 |
| 8 | Y | 5.4 | 5.9 | 5.4 | 5.7 | 0 | 6 | 5.8 | 5.7 | 0 |
| 8 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | Y | 0.6 | 0.6 | 0.6 | 0.6 | 0 | 0.6 | 0.6 | 0.6 | 5.7 |
| 9 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | Y | 1.3 | 1.3 | 1.6 | 1.5 | 1.5 | 1.3 | 1.7 | 1.3 | 1 |
| 10 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | Y | 0 | 0 | 0 | 0 | 3.6 | 0 | 0 | 0 | 0 |
| 11 | Z | 0 | 41.6 | 0 | 0 | 0 | 41.6 | 0 | 41.6 | 12.9 |

Table 1-1: Participation factor per mode scaled to unit generalised mass [√kg], global X/Y/Z — shell, order 1.

| Mode | Direction | ses_QUAD | ses_TRI | aba_QUADR | aba_QUAD | ccx_TRI | ca_TRI | ccx_QUAD | ca_QUAD | ccx_QUADR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | -0 | -0 | -0 | -0 | 0 | -0 | 0 | 0 | -0 |
| 1 | Y | 10.95 | -10.951 | 10.952 | 10.951 | 10.941 | 10.951 | 10.95 | 10.951 | 10.95 |
| 1 | Z | -0 | -0 | -0 | -0 | 0 | 0 | 0 | -0 | 0 |
| 2 | X | -0 | 0 | -0 | -0 | -0 | 0 | 0 | -0 | -0 |
| 2 | Y | -0 | -0 | 0 | 0 | 0.002 | -0 | 0 | -0 | 0 |
| 2 | Z | -0 | 0 | 0 | 0 | -0 | 0 | 0 | 0 | -11.051 |
| 3 | X | 0 | -0 | -0 | -0 | -0 | 0 | -0 | -0 | -0 |
| 3 | Y | -0 | 0 | 0 | 0 | 0 | -0 | 0 | 0 | 6.101 |
| 3 | Z | -11.046 | -11.047 | 11.046 | 11.046 | -11.048 | 11.047 | -11.045 | 11.046 | 0 |
| 4 | X | -0 | 0 | -0 | -0 | -0 | 0 | -0 | -0 | 0 |
| 4 | Y | -6.067 | -6.048 | -6.081 | -6.076 | -6.104 | -6.053 | -6.072 | 6.065 | 0 |
| 4 | Z | -0 | -0 | -0 | -0 | 0 | -0 | 0 | 0 | 0 |
| 5 | X | 0 | -0 | 0 | -0 | 0 | 0 | -0 | -0 | -0 |
| 5 | Y | 0 | 0 | -0 | -0 | -0.005 | -0 | -0 | 0 | -3.601 |
| 5 | Z | -0 | -0 | 0 | -0 | -0 | -0 | 0 | -0 | 0 |
| 6 | X | -0 | -0 | 0 | 0 | -0 | -0 | 0 | -0 | 0 |
| 6 | Y | 0.134 | -0.252 | -0.078 | -0.132 | -3.561 | 0.263 | -0.174 | -0.194 | -0 |
| 6 | Z | -0 | 0 | -0 | -0 | -0 | -0 | -0 | -0 | -6.443 |
| 7 | X | 0 | -0 | -0 | -0 | -0 | -0 | -0 | 0 | -0 |
| 7 | Y | -1.86 | -1.651 | 1.947 | 1.829 | -0 | -1.652 | 1.805 | 1.776 | -0 |
| 7 | Z | 0 | 0 | 0 | 0 | -6.446 | 0 | 0 | 0 | 0 |
| 8 | X | 0 | -0 | 0 | -0 | 0 | 0 | -0 | -0 | 12.568 |
| 8 | Y | 2.334 | 2.438 | 2.332 | 2.383 | 0.003 | -2.455 | -2.406 | 2.398 | 0 |
| 8 | Z | 0 | -0 | -0 | -0 | 0 | -0 | 0 | -0 | 0 |
| 9 | X | -0 | -0 | -0 | 0 | -0 | -0 | -0 | -0 | -0 |
| 9 | Y | -0.798 | -0.781 | -0.786 | -0.771 | -0.156 | 0.788 | -0.76 | -0.795 | -2.388 |
| 9 | Z | 0 | -0 | 0 | 0 | 0 | -0 | 0 | -0 | -0 |
| 10 | X | 0 | 0 | -0 | 0 | -0 | 0 | -0 | -0 | -0 |
| 10 | Y | -1.15 | -1.128 | 1.269 | 1.232 | 1.218 | -1.148 | 1.31 | 1.152 | 1.006 |
| 10 | Z | 0 | -0 | 0 | 0 | -0 | 0 | 0 | -0 | -0 |
| 11 | X | 0 | -0 | -0 | 0 | -0 | 0 | 0 | 0 | -0 |
| 11 | Y | -0 | -0 | 0 | 0 | -1.9 | -0 | 0 | 0 | 0 |
| 11 | Z | 0 | -6.451 | 0 | 0 | 0 | 6.453 | 0 | 6.453 | 3.589 |

Shell, 2nd order

Table 1-1: Effective modal mass [kg] per mode, global X/Y/Z — shell, order 2.

| Mode | Direction | ses_QUAD | ses_TRI | aba_QUADR | aba_QUAD | aba_TRI | ccx_TRI | ca_TRI | ccx_QUAD | ca_QUAD | ccx_QUADR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | Y | 119.9 | 119.9 | 119.9 | 119.9 | 119.9 | 119.9 | 119.9 | 119.9 | 119.9 | 119.9 |
| 1 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | Z | 122 | 122 | 122 | 122 | 122 | 122 | 122 | 122 | 122 | 122 |
| 4 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | Y | 36.9 | 36.9 | 36.9 | 36.9 | 36.9 | 36.9 | 36.9 | 36.9 | 36.9 | 36.9 |
| 4 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | Y | 3.5 | 3.4 | 3.5 | 3.5 | 3.5 | 3.4 | 3.4 | 3.4 | 3.5 | 3.4 |
| 7 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | Y | 5.5 | 5.6 | 5.5 | 5.5 | 5.5 | 5.6 | 5.5 | 5.6 | 5.5 | 5.5 |
| 8 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | Y | 0.7 | 0.6 | 0.7 | 0.7 | 0.7 | 0.6 | 0.6 | 0.6 | 0.7 | 0.6 |
| 9 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | Y | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 |
| 10 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | Y | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11 | Z | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 1-1: Participation factor per mode scaled to unit generalised mass [√kg], global X/Y/Z — shell, order 2.

| Mode | Direction | ses_QUAD | ses_TRI | aba_QUADR | aba_QUAD | aba_TRI | ccx_TRI | ca_TRI | ccx_QUAD | ca_QUAD | ccx_QUADR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | -0 | -0 | 0 | 0 | 0 | -0 | 0 | 0 | -0 | -0 |
| 1 | Y | -10.952 | 10.952 | 10.951 | 10.951 | 10.952 | 10.952 | 10.952 | 10.951 | 10.952 | 10.951 |
| 1 | Z | -0 | -0 | 0 | 0 | 0 | 0 | -0 | 0 | 0 | 0 |
| 2 | X | -0 | 0 | 0 | 0 | 0 | -0 | -0 | 0 | 0 | 0 |
| 2 | Y | 0 | 0 | 0 | 0 | 0 | 0 | -0 | 0 | -0 | 0 |
| 2 | Z | 0 | 0 | 0 | 0 | -0 | -0 | 0 | -0 | -0 | -0 |
| 3 | X | -0 | -0 | 0 | 0 | 0 | -0 | 0 | 0 | 0 | 0 |
| 3 | Y | -0 | -0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | Z | 11.047 | -11.047 | 11.044 | 11.044 | 11.047 | -11.046 | 11.047 | -11.046 | 11.047 | -11.046 |
| 4 | X | -0 | -0 | -0 | -0 | 0 | -0 | -0 | -0 | 0 | 0 |
| 4 | Y | -6.071 | 6.072 | -6.071 | -6.071 | -6.073 | 6.072 | 6.071 | 6.073 | -6.071 | 6.071 |
| 4 | Z | 0 | -0 | 0 | 0 | 0 | -0 | -0 | -0 | 0 | -0 |
| 5 | X | 0 | -0 | -0 | -0 | -0 | 0 | -0 | -0 | 0 | -0 |
| 5 | Y | 0 | 0 | -0 | -0 | -0 | 0 | -0 | -0 | -0 | -0 |
| 5 | Z | -0 | 0 | -0 | -0 | 0 | -0 | -0 | -0 | -0 | 0 |
| 6 | X | -0 | 0 | -0 | -0 | -0 | -0 | 0 | -0 | 0 | 0 |
| 6 | Y | -0.137 | -0.144 | -0.137 | -0.137 | -0.138 | -0.144 | 0.142 | 0.141 | -0.137 | 0.138 |
| 6 | Z | 0 | 0 | 0 | 0 | -0 | -0 | -0 | 0 | 0 | 0 |
| 7 | X | 0 | -0 | -0 | -0 | -0 | -0 | -0 | -0 | -0 | -0 |
| 7 | Y | 1.859 | -1.85 | 1.858 | 1.858 | 1.88 | 1.848 | -1.849 | 1.839 | -1.859 | 1.852 |
| 7 | Z | -0 | 0 | -0 | -0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | X | -0 | 0 | -0 | -0 | -0 | 0 | -0 | -0 | -0 | 0 |
| 8 | Y | -2.35 | -2.358 | 2.351 | 2.351 | 2.349 | 2.356 | 2.353 | -2.362 | 2.35 | -2.35 |
| 8 | Z | -0 | -0 | 0 | 0 | 0 | -0 | 0 | 0 | 0 | 0 |
| 9 | X | 0 | 0 | -0 | -0 | 0 | -0 | -0 | 0 | -0 | 0 |
| 9 | Y | -0.809 | -0.801 | -0.808 | -0.808 | -0.818 | 0.793 | 0.801 | 0.781 | -0.809 | 0.799 |
| 9 | Z | 0 | 0 | 0 | 0 | -0 | -0 | 0 | -0 | -0 | -0 |
| 10 | X | -0 | -0 | -0 | -0 | -0 | -0 | 0 | -0 | -0 | -0 |
| 10 | Y | -1.171 | -1.185 | 1.17 | 1.17 | 1.175 | 1.193 | 1.178 | -1.201 | 1.17 | 1.178 |
| 10 | Z | -0 | -0 | -0 | -0 | 0.001 | 0 | 0 | -0 | 0 | 0 |
| 11 | X | -0 | -0 | 0 | 0 | 0 | -0 | -0 | -0 | -0 | -0 |
| 11 | Y | -0 | 0.001 | 0 | 0 | 0 | 0.001 | -0.001 | 0 | 0 | 0 |
| 11 | Z | 0 | -0 | -0 | -0 | 0 | -0 | -0 | 0 | -0 | -0 |

Line, 1st order

Table 1-1: Effective modal mass [kg] per mode, global X/Y/Z — line, order 1.

| Mode | Direction | ses | aba | ca |
| --- | --- | --- | --- | --- |
| 1 | X | 0 | 0 | 0 |
| 1 | Y | 0 | 0 | 0 |
| 1 | Z | 0 | 0 | 0 |
| 2 | X | 0 | 0 | 0 |
| 2 | Y | 116.7 | 116.6 | 116.5 |
| 2 | Z | 0 | 0 | 0 |
| 3 | X | 0 | 0 | 0 |
| 3 | Y | 0 | 0 | 0 |
| 3 | Z | 0 | 0 | 0 |
| 4 | X | 0 | 0 | 0 |
| 4 | Y | 0 | 0 | 0 |
| 4 | Z | 128 | 118.2 | 116.5 |
| 5 | X | 0 | 0 | 0 |
| 5 | Y | 0 | 0 | 0 |
| 5 | Z | 0 | 0 | 0 |
| 6 | X | 0 | 0 | 0 |
| 6 | Y | 36.3 | 0 | 35.8 |
| 6 | Z | 0 | 0 | 0 |
| 7 | X | 0 | 0 | 0 |
| 7 | Y | 0 | 36.2 | 0 |
| 7 | Z | 0 | 0 | 0 |
| 8 | X | 0 | 0 | 0 |
| 8 | Y | 0 | 0 | 0 |
| 8 | Z | 0 | 0 | 0 |
| 9 | X | 0 | 0 | 0 |
| 9 | Y | 0 | 0 | 0 |
| 9 | Z | 0 | 0 | 0 |
| 10 | X | 0 | 0 | 0 |
| 10 | Y | 0 | 0 | 0 |
| 10 | Z | 0 | 0 | 0 |
| 11 | X | 0 | 0 | 0 |
| 11 | Y | 0 | 0 | 0 |
| 11 | Z | 38.3 | 0 | 0 |

Table 1-1: Participation factor per mode scaled to unit generalised mass [√kg], global X/Y/Z — line, order 1.

| Mode | Direction | ses | aba | ca |
| --- | --- | --- | --- | --- |
| 1 | X | 0 | -0 | -0 |
| 1 | Y | 0 | -0 | 0 |
| 1 | Z | -0 | 0 | 0 |
| 2 | X | 0 | -0 | 0 |
| 2 | Y | 10.803 | 10.797 | 10.793 |
| 2 | Z | -0 | -0 | 0 |
| 3 | X | -0 | 0 | -0 |
| 3 | Y | 0 | -0 | -0 |
| 3 | Z | -0 | 0 | -0 |
| 4 | X | -0 | 0 | -0 |
| 4 | Y | 0 | -0 | -0 |
| 4 | Z | -11.316 | 10.87 | 10.793 |
| 5 | X | -0 | -0 | 0 |
| 5 | Y | -0 | 0 | 0 |
| 5 | Z | 0 | 0 | 0 |
| 6 | X | -0 | -0 | -0 |
| 6 | Y | -6.024 | -0 | -5.981 |
| 6 | Z | -0 | 0 | -0 |
| 7 | X | -0 | 0 | -0 |
| 7 | Y | -0 | -6.016 | -0 |
| 7 | Z | -0 | -0 | -0 |
| 8 | X | 0 | -0 | -0 |
| 8 | Y | 0 | -0 | -0 |
| 8 | Z | -0 | 0 | 0 |
| 9 | X | -0 | 0 | -0 |
| 9 | Y | 0 | 0 | 0 |
| 9 | Z | -0 | 0 | -0 |
| 10 | X | -0 | 0 | -0 |
| 10 | Y | 0 | 0 | 0 |
| 10 | Z | -0 | -0 | 0 |
| 11 | X | -0 | 0 | -0 |
| 11 | Y | 0 | -0 | 0 |
| 11 | Z | -6.193 | 0 | 0 |

Line, 2nd order

Table 1-1: Effective modal mass [kg] per mode, global X/Y/Z — line, order 2.

| Mode | Direction | ses | aba |
| --- | --- | --- | --- |
| 1 | X | 0 | 0 |
| 1 | Y | 0 | 0 |
| 1 | Z | 0 | 0 |
| 2 | X | 0 | 0 |
| 2 | Y | 176.8 | 116.6 |
| 2 | Z | 0 | 0 |
| 3 | X | 0 | 0 |
| 3 | Y | 0 | 0 |
| 3 | Z | 0 | 0 |
| 4 | X | 0 | 0 |
| 4 | Y | 0 | 0 |
| 4 | Z | 192.8 | 118.2 |
| 5 | X | 0 | 0 |
| 5 | Y | 0 | 0 |
| 5 | Z | 0 | 0 |
| 6 | X | 0 | 0 |
| 6 | Y | 55 | 0 |
| 6 | Z | 0 | 0 |
| 7 | X | 0 | 0 |
| 7 | Y | 0 | 36.2 |
| 7 | Z | 0 | 0 |
| 8 | X | 0 | 0 |
| 8 | Y | 0 | 0 |
| 8 | Z | 0 | 0 |
| 9 | X | 0 | 0 |
| 9 | Y | 0 | 0 |
| 9 | Z | 0 | 0 |
| 10 | X | 0 | 0 |
| 10 | Y | 0 | 0 |
| 10 | Z | 0 | 0 |
| 11 | X | 0 | 0 |
| 11 | Y | 0 | 0 |
| 11 | Z | 59.6 | 0 |

Table 1-1: Participation factor per mode scaled to unit generalised mass [√kg], global X/Y/Z — line, order 2.

| Mode | Direction | ses | aba |
| --- | --- | --- | --- |
| 1 | X | 0 | 0 |
| 1 | Y | 0 | 0 |
| 1 | Z | -0 | -0 |
| 2 | X | -0 | -0 |
| 2 | Y | 13.297 | 10.798 |
| 2 | Z | 0 | -0 |
| 3 | X | 0 | 0 |
| 3 | Y | 0 | -0 |
| 3 | Z | -0 | -0 |
| 4 | X | 0 | -0 |
| 4 | Y | 0 | -0 |
| 4 | Z | -13.884 | 10.871 |
| 5 | X | -0 | -0 |
| 5 | Y | -0 | 0 |
| 5 | Z | -0 | 0 |
| 6 | X | 0 | -0 |
| 6 | Y | -7.413 | 0 |
| 6 | Z | -0 | -0 |
| 7 | X | -0 | 0 |
| 7 | Y | 0 | -6.015 |
| 7 | Z | 0 | -0 |
| 8 | X | -0 | 0 |
| 8 | Y | 0 | 0 |
| 8 | Z | 0 | -0 |
| 9 | X | 0 | 0 |
| 9 | Y | -0 | 0 |
| 9 | Z | -0 | -0 |
| 10 | X | 0 | -0 |
| 10 | Y | 0 | 0 |
| 10 | Z | -0 | 0 |
| 11 | X | 0 | -0 |
| 11 | Y | 0 | -0 |
| 11 | Z | -7.719 | 0 |

Cantilever eigenvalue analysis

Line, 1st order

- Mode 1

Figure 1: Abaqus — mode 1, 10.785 Hz

Figure 2: Sesam — mode 1, 12.245 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 10.785 | -6.34 | 4592.3 | 4.83e-34 | 1.00e-29 | 1.05e-31 | -2.20e-17 | -3.17e-15 | 3.24e-16 |
| Sesam | 12.245 | +6.34 | 5919.77 | 8.88e-35 | 2.77e-27 | 1.18e-30 | 9.42e-18 | 5.26e-14 | -1.09e-15 |

Mode 2

Figure 3: Abaqus — mode 2, 12.928 Hz

Figure 4: Sesam — mode 2, 12.943 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.928 | -0.06 | 6598.4 | 6.75e-31 | 116.57 | 3.27e-31 | -8.21e-16 | 10.797 | -5.72e-16 |
| Sesam | 12.943 | +0.06 | 6613.55 | 4.64e-35 | 116.71 | 5.64e-31 | 6.81e-18 | 10.803 | -7.51e-16 |

Mode 3

Figure 5: Abaqus — mode 3, 32.342 Hz

Figure 6: Sesam — mode 3, 36.752 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 32.342 | -6.38 | 41294 | 7.85e-31 | 8.23e-32 | 1.14e-28 | 8.86e-16 | -2.87e-16 | 1.07e-14 |
| Sesam | 36.752 | +6.38 | 53325.3 | 8.68e-35 | 1.01e-30 | 1.52e-32 | -9.32e-18 | 1.00e-15 | -1.23e-16 |

Mode 4

Figure 7: Abaqus — mode 4, 50.543 Hz

Figure 8: Sesam — mode 4, 44.962 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 50.543 | +5.84 | 100852 | 2.51e-30 | 1.42e-29 | 118.16 | 1.58e-15 | -3.76e-15 | 10.870 |
| Sesam | 44.962 | -5.84 | 79809.7 | 3.16e-33 | 9.04e-30 | 128.05 | -5.62e-17 | 3.01e-15 | -11.316 |

Mode 5

Figure 9: Abaqus — mode 5, 53.855 Hz

Figure 10: Sesam — mode 5, 61.309 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 53.855 | -6.47 | 114501 | 1.75e-29 | 8.90e-30 | 2.61e-27 | -4.18e-15 | 2.98e-15 | 5.11e-14 |
| Sesam | 61.309 | +6.47 | 148390 | 3.02e-32 | 3.54e-30 | 1.09e-29 | -1.74e-16 | -1.88e-15 | 3.30e-15 |

Mode 6

Figure 11: Abaqus — mode 6, 75.296 Hz

Figure 12: Sesam — mode 6, 79.793 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 75.296 | -2.90 | 223823 | 3.82e-29 | 2.17e-28 | 3.67e-28 | -6.18e-15 | -1.47e-14 | 1.92e-14 |
| Sesam | 79.793 | +2.90 | 251356 | 8.03e-33 | 36.29 | 4.50e-29 | -8.96e-17 | -6.024 | -6.71e-15 |

Mode 7

Figure 13: Abaqus — mode 7, 79.590 Hz

Figure 14: Sesam — mode 7, 85.947 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 79.590 | -3.84 | 250078 | 1.01e-30 | 36.20 | 1.38e-29 | 1.01e-15 | -6.016 | -3.72e-15 |
| Sesam | 85.947 | +3.84 | 291620 | 1.09e-31 | 9.28e-29 | 2.34e-30 | -3.31e-16 | -9.63e-15 | -1.53e-15 |

Mode 8

Figure 15: Abaqus — mode 8, 96.637 Hz

Figure 16: Sesam — mode 8, 110.699 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 96.637 | -6.78 | 368677 | 4.04e-26 | 1.03e-27 | 4.21e-27 | -2.01e-13 | -3.20e-14 | 6.49e-14 |
| Sesam | 110.699 | +6.78 | 483782 | 2.49e-33 | 6.18e-30 | 2.43e-29 | 4.99e-17 | 2.49e-15 | -4.93e-15 |

Mode 9

Figure 17: Abaqus — mode 9, 117.849 Hz

Figure 18: Sesam — mode 9, 135.600 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 117.850 | -7.00 | 548290 | 9.24e-23 | 1.72e-25 | 7.37e-24 | 9.61e-12 | 4.15e-13 | 2.71e-12 |
| Sesam | 135.600 | +7.00 | 725900 | 3.52e-30 | 2.31e-30 | 4.44e-29 | -1.88e-15 | 1.52e-15 | -6.66e-15 |

Mode 10

Figure 19: Abaqus — mode 10, 138.904 Hz

Figure 20: Sesam — mode 10, 160.681 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 138.900 | -7.27 | 761704 | 1.00e-20 | 2.09e-22 | 2.61e-21 | 1.00e-10 | 1.45e-11 | -5.11e-11 |
| Sesam | 160.681 | +7.27 | 1.01927e+06 | 6.22e-23 | 1.67e-23 | 7.04e-23 | -7.88e-12 | 4.09e-12 | -8.39e-12 |

Mode 11

Figure 21: Abaqus — mode 11, 159.773 Hz

Figure 22: Sesam — mode 11, 176.495 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 159.770 | -4.97 | 1.00778e+06 | 1.05e-18 | 1.96e-22 | 2.87e-21 | 1.02e-09 | -1.40e-11 | 5.35e-11 |
| Sesam | 176.495 | +4.97 | 1.22977e+06 | 3.95e-21 | 1.04e-21 | 38.35 | -6.28e-11 | 3.22e-11 | -6.193 |

Line, 2nd order

Mode 1

Figure 23: Abaqus — mode 1, 10.786 Hz

Figure 24: Sesam — mode 1, 10.039 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 10.786 | +3.59 | 4592.8 | 3.02e-31 | 1.31e-30 | 3.09e-30 | 5.50e-16 | 1.14e-15 | -1.76e-15 |
| Sesam | 10.039 | -3.59 | 3978.68 | 5.26e-40 | 1.31e-27 | 4.46e-29 | 2.29e-20 | 3.62e-14 | -6.68e-15 |

Mode 2

Figure 25: Abaqus — mode 2, 12.932 Hz

Figure 26: Sesam — mode 2, 10.614 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.932 | +9.84 | 6602.4 | 1.50e-32 | 116.59 | 2.60e-32 | -1.22e-16 | 10.798 | -1.61e-16 |
| Sesam | 10.614 | -9.84 | 4447.85 | 3.31e-28 | 176.82 | 7.82e-31 | -1.82e-14 | 13.297 | 8.84e-16 |

Mode 3

Figure 27: Abaqus — mode 3, 32.358 Hz

Figure 28: Sesam — mode 3, 30.108 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 32.358 | +3.60 | 41335 | 2.64e-31 | 5.69e-29 | 8.91e-29 | 5.14e-16 | -7.54e-15 | -9.44e-15 |
| Sesam | 30.108 | -3.60 | 35786.3 | 4.57e-37 | 6.51e-30 | 2.81e-29 | 6.76e-19 | 2.55e-15 | -5.30e-15 |

Mode 4

Figure 29: Abaqus — mode 4, 50.557 Hz

Figure 30: Sesam — mode 4, 36.752 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 50.557 | +15.81 | 100907 | 6.29e-36 | 9.63e-28 | 118.18 | -2.51e-18 | -3.10e-14 | 10.871 |
| Sesam | 36.752 | -15.81 | 53323.9 | 3.71e-28 | 2.58e-29 | 192.77 | 1.93e-14 | 5.08e-15 | -13.884 |

Mode 5

Figure 31: Abaqus — mode 5, 53.930 Hz

Figure 32: Sesam — mode 5, 50.149 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 53.930 | +3.63 | 114820 | 1.13e-29 | 6.97e-31 | 2.94e-28 | -3.36e-15 | 8.35e-16 | 1.72e-14 |
| Sesam | 50.149 | -3.63 | 99284.4 | 1.91e-35 | 1.10e-30 | 4.31e-29 | -4.37e-18 | -1.05e-15 | -6.57e-15 |

Mode 6

Figure 33: Abaqus — mode 6, 75.502 Hz

Figure 34: Sesam — mode 6, 65.327 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 75.502 | +7.23 | 225048 | 6.44e-34 | 3.07e-27 | 1.85e-29 | -2.54e-17 | 5.54e-14 | -4.30e-15 |
| Sesam | 65.327 | -7.23 | 168476 | 1.08e-28 | 54.95 | 1.07e-28 | 1.04e-14 | -7.413 | -1.03e-14 |

Mode 7

Figure 35: Abaqus — mode 7, 79.684 Hz

Figure 36: Sesam — mode 7, 70.143 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 79.684 | +6.37 | 250668 | 9.57e-31 | 36.19 | 1.97e-27 | 9.78e-16 | -6.015 | -4.44e-14 |
| Sesam | 70.143 | -6.37 | 194235 | 7.33e-35 | 4.66e-28 | 4.83e-30 | -8.56e-18 | 2.16e-14 | 2.20e-15 |

Mode 8

Figure 37: Abaqus — mode 8, 97.074 Hz

Figure 38: Sesam — mode 8, 90.070 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 97.074 | +3.74 | 372022 | 4.46e-28 | 1.86e-30 | 1.09e-27 | 2.11e-14 | 1.36e-15 | -3.31e-14 |
| Sesam | 90.070 | -3.74 | 320270 | 1.65e-33 | 4.85e-29 | 3.89e-29 | -4.06e-17 | 6.96e-15 | 6.24e-15 |

Mode 9

Figure 39: Abaqus — mode 9, 118.648 Hz

Figure 40: Sesam — mode 9, 109.907 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 118.650 | +3.83 | 555748 | 3.94e-25 | 6.15e-26 | 2.26e-25 | 6.28e-13 | 2.48e-13 | -4.75e-13 |
| Sesam | 109.907 | -3.83 | 476881 | 3.54e-29 | 2.19e-32 | 8.17e-29 | 5.95e-15 | -1.48e-16 | -9.04e-15 |

Mode 10

Figure 41: Abaqus — mode 10, 140.222 Hz

Figure 42: Sesam — mode 10, 129.631 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 140.220 | +3.92 | 776236 | 2.58e-24 | 4.27e-23 | 1.99e-23 | -1.61e-12 | 6.54e-12 | 4.46e-12 |
| Sesam | 129.631 | -3.92 | 663407 | 8.02e-21 | 1.12e-21 | 1.11e-21 | 8.95e-11 | 3.35e-11 | -3.34e-11 |

Mode 11

Figure 43: Abaqus — mode 11, 161.799 Hz

Figure 44: Sesam — mode 11, 142.572 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 161.800 | +6.32 | 1.03351e+06 | 2.30e-19 | 1.18e-19 | 3.67e-20 | -4.80e-10 | -3.43e-10 | 1.92e-10 |
| Sesam | 142.572 | -6.32 | 802475 | 5.07e-20 | 7.21e-21 | 59.58 | 2.25e-10 | 8.49e-11 | -7.719 |

Shell, 1st order, TRI

Mode 1

Figure 45: Sesam — mode 1, 13.641 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 13.641 | 7346.4 | 2.57e-27 | 119.93 | 7.93e-25 | -5.07e-14 | -10.951 | -8.90e-13 |

Mode 2

Figure 46: Sesam — mode 2, 20.198 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 20.198 | 16106.1 | 1.92e-26 | 3.02e-11 | 1.82e-25 | 1.38e-13 | -5.49e-06 | 4.26e-13 |

Mode 3

Figure 47: Sesam — mode 3, 52.192 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 52.192 | 107540 | 4.79e-10 | 1.24e-24 | 122.03 | -2.19e-05 | 1.12e-12 | -11.047 |

Mode 4

Figure 48: Sesam — mode 4, 79.523 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 79.523 | 249655 | 1.26e-27 | 36.58 | 1.11e-25 | 3.55e-14 | -6.048 | -3.33e-13 |

Mode 5

Figure 49: Sesam — mode 5, 101.756 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 101.756 | 408770 | 3.14e-27 | 4.27e-09 | 1.42e-27 | -5.60e-14 | 6.54e-05 | -3.77e-14 |

Mode 6

Figure 50: Sesam — mode 6, 125.093 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 125.093 | 617772 | 5.83e-28 | 0.06 | 2.58e-27 | -2.41e-14 | -0.252 | 5.08e-14 |

Mode 7

Figure 51: Sesam — mode 7, 143.164 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 143.164 | 809150 | 1.19e-28 | 2.72 | 6.22e-28 | -1.09e-14 | -1.651 | 2.49e-14 |

Mode 8

Figure 52: Sesam — mode 8, 166.668 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 166.668 | 1.09664e+06 | 1.64e-27 | 5.94 | 3.89e-27 | -4.05e-14 | 2.438 | -6.23e-14 |

Mode 9

Figure 53: Sesam — mode 9, 191.533 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 191.533 | 1.44826e+06 | 2.66e-28 | 0.61 | 7.55e-28 | -1.63e-14 | -0.781 | -2.75e-14 |

Mode 10

Figure 54: Sesam — mode 10, 230.489 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 230.489 | 2.09731e+06 | 2.70e-28 | 1.27 | 2.42e-28 | 1.64e-14 | -1.128 | -1.55e-14 |

Mode 11

Figure 55: Sesam — mode 11, 257.362 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sesam | 257.362 | 2.61487e+06 | 6.51e-09 | 3.57e-22 | 41.62 | -8.07e-05 | -1.89e-11 | -6.451 |

Shell, 1st order, QUAD

Mode 1

Figure 56: Abaqus — mode 1, 12.780 Hz

Figure 57: Sesam — mode 1, 12.763 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.780 | +0.07 | 6448.4 | 7.41e-27 | 119.92 | 1.38e-25 | -8.61e-14 | 10.951 | -3.71e-13 |
| Sesam | 12.763 | -0.07 | 6430.64 | 1.47e-28 | 119.90 | 2.22e-25 | -1.21e-14 | 10.950 | -4.71e-13 |

Mode 2

Figure 58: Abaqus — mode 2, 19.593 Hz

Figure 59: Sesam — mode 2, 19.394 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 19.593 | +0.51 | 15155 | 1.46e-27 | 1.08e-19 | 7.67e-28 | -3.83e-14 | 3.28e-10 | 2.77e-14 |
| Sesam | 19.394 | -0.51 | 14849.3 | 3.80e-27 | 7.43e-19 | 1.60e-24 | -6.16e-14 | -8.62e-10 | -1.27e-12 |

Mode 3

Figure 60: Abaqus — mode 3, 51.998 Hz

Figure 61: Sesam — mode 3, 51.991 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 51.998 | +0.01 | 106741 | 1.35e-26 | 1.05e-26 | 122.01 | -1.16e-13 | 1.03e-13 | 11.046 |
| Sesam | 51.991 | -0.01 | 106714 | 7.03e-28 | 8.47e-26 | 122.00 | 2.65e-14 | -2.91e-13 | -11.046 |

Mode 4

Figure 62: Abaqus — mode 4, 75.683 Hz

Figure 63: Sesam — mode 4, 75.192 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 75.683 | +0.33 | 226132 | 1.88e-28 | 36.92 | 2.68e-25 | -1.37e-14 | -6.076 | -5.18e-13 |
| Sesam | 75.192 | -0.33 | 223203 | 1.10e-27 | 36.80 | 9.74e-26 | -3.31e-14 | -6.067 | -3.12e-13 |

Mode 5

Figure 64: Abaqus — mode 5, 96.393 Hz

Figure 65: Sesam — mode 5, 95.930 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 96.393 | +0.24 | 366818 | 7.25e-28 | 1.11e-22 | 7.70e-26 | -2.69e-14 | -1.05e-11 | -2.77e-13 |
| Sesam | 95.930 | -0.24 | 363300 | 2.85e-27 | 3.92e-24 | 2.81e-26 | 5.34e-14 | 1.98e-12 | -1.68e-13 |

Mode 6

Figure 66: Abaqus — mode 6, 126.961 Hz

Figure 67: Sesam — mode 6, 125.116 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 126.960 | +0.73 | 636351 | 1.52e-27 | 0.02 | 2.42e-26 | 3.90e-14 | -0.132 | -1.56e-13 |
| Sesam | 125.116 | -0.73 | 617993 | 3.59e-30 | 0.02 | 5.54e-26 | -1.89e-15 | 0.134 | -2.35e-13 |

Mode 7

Figure 68: Abaqus — mode 7, 145.566 Hz

Figure 69: Sesam — mode 7, 141.556 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 145.570 | +1.40 | 836523 | 2.58e-27 | 3.34 | 1.12e-26 | -5.08e-14 | 1.829 | 1.06e-13 |
| Sesam | 141.556 | -1.40 | 791068 | 1.14e-28 | 3.46 | 3.39e-27 | 1.07e-14 | -1.860 | 5.82e-14 |

Mode 8

Figure 70: Abaqus — mode 8, 169.389 Hz

Figure 71: Sesam — mode 8, 165.030 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 169.390 | +1.30 | 1.13275e+06 | 2.66e-25 | 5.68 | 6.71e-24 | -5.16e-13 | 2.383 | -2.59e-12 |
| Sesam | 165.030 | -1.30 | 1.07519e+06 | 5.28e-28 | 5.45 | 4.78e-26 | 2.30e-14 | 2.334 | 2.19e-13 |

Mode 9

Figure 72: Abaqus — mode 9, 198.748 Hz

Figure 73: Sesam — mode 9, 190.099 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 198.750 | +2.22 | 1.55943e+06 | 6.12e-22 | 0.59 | 1.15e-20 | 2.47e-11 | -0.771 | 1.07e-10 |
| Sesam | 190.099 | -2.22 | 1.42665e+06 | 1.59e-30 | 0.64 | 8.33e-27 | -1.26e-15 | -0.798 | 9.13e-14 |

Mode 10

Figure 74: Abaqus — mode 10, 242.545 Hz

Figure 75: Sesam — mode 10, 229.460 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 242.550 | +2.77 | 2.32244e+06 | 3.62e-17 | 1.52 | 5.68e-16 | 6.02e-09 | 1.232 | 2.38e-08 |
| Sesam | 229.460 | -2.77 | 2.07861e+06 | 1.61e-29 | 1.32 | 5.25e-27 | 4.01e-15 | -1.150 | 7.25e-14 |

Mode 11

Figure 76: Abaqus — mode 11, 252.486 Hz

Figure 77: Sesam — mode 11, 250.786 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 252.490 | +0.34 | 2.51672e+06 | 4.88e-17 | 7.27e-17 | 7.10e-16 | 6.99e-09 | 8.53e-09 | 2.67e-08 |
| Sesam | 250.786 | -0.34 | 2.48294e+06 | 6.33e-21 | 7.49e-20 | 8.71e-20 | 7.95e-11 | -2.74e-10 | 2.95e-10 |

Shell, 1st order, QUAD, reduced integration

Mode 1

Figure 78: Abaqus — mode 1, 12.373 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.373 | 6043.5 | 1.78e-28 | 119.95 | 5.35e-25 | -1.33e-14 | 10.952 | -7.31e-13 |

Mode 2

Figure 79: Abaqus — mode 2, 19.230 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 19.230 | 14599 | 1.25e-26 | 4.60e-21 | 3.10e-26 | -1.12e-13 | 6.78e-11 | 1.76e-13 |

Mode 3

Figure 80: Abaqus — mode 3, 51.849 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 51.849 | 106129 | 2.48e-26 | 1.19e-25 | 122.02 | -1.58e-13 | 3.45e-13 | 11.046 |

Mode 4

Figure 81: Abaqus — mode 4, 73.558 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 73.558 | 213611 | 2.90e-28 | 36.98 | 6.94e-25 | -1.70e-14 | -6.081 | -8.33e-13 |

Mode 5

Figure 82: Abaqus — mode 5, 93.709 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 93.709 | 346673 | 9.12e-28 | 8.02e-23 | 5.37e-26 | 3.02e-14 | -8.96e-12 | 2.32e-13 |

Mode 6

Figure 83: Abaqus — mode 6, 126.870 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 126.870 | 635447 | 1.80e-30 | 0.01 | 1.84e-25 | 1.34e-15 | -0.078 | -4.29e-13 |

Mode 7

Figure 84: Abaqus — mode 7, 144.579 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 144.580 | 825226 | 1.99e-29 | 3.79 | 2.06e-26 | -4.46e-15 | 1.947 | 1.43e-13 |

Mode 8

Figure 85: Abaqus — mode 8, 168.379 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 168.380 | 1.11927e+06 | 2.51e-28 | 5.44 | 1.97e-25 | 1.58e-14 | 2.332 | -4.43e-13 |

Mode 9

Figure 86: Abaqus — mode 9, 197.515 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 197.510 | 1.54014e+06 | 1.19e-26 | 0.62 | 1.84e-25 | -1.09e-13 | -0.786 | 4.29e-13 |

Mode 10

Figure 87: Abaqus — mode 10, 241.275 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 241.270 | 2.29818e+06 | 2.67e-22 | 1.61 | 1.69e-20 | -1.63e-11 | 1.269 | 1.30e-10 |

Mode 11

Figure 88: Abaqus — mode 11, 245.071 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 245.070 | 2.37106e+06 | 3.55e-23 | 8.35e-21 | 8.47e-21 | -5.95e-12 | 9.14e-11 | 9.20e-11 |

Shell, 2nd order, TRI

Mode 1

Figure 89: Abaqus — mode 1, 12.756 Hz

Figure 90: Sesam — mode 1, 12.756 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.756 | 9.02e-04 | 6423.3 | 2.07e-18 | 119.94 | 3.02e-12 | 1.44e-09 | 10.952 | 1.74e-06 |
| Sesam | 12.756 | -9.02e-04 | 6423.52 | 1.73e-27 | 119.94 | 5.84e-24 | -4.16e-14 | 10.952 | -2.42e-12 |

Mode 2

Figure 91: Abaqus — mode 2, 19.567 Hz

Figure 92: Sesam — mode 2, 19.505 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 19.567 | +0.16 | 15115 | 3.71e-13 | 3.08e-10 | 4.41e-12 | 6.09e-07 | 1.75e-05 | -2.10e-06 |
| Sesam | 19.505 | -0.16 | 15019.1 | 2.03e-25 | 1.78e-08 | 1.33e-23 | 4.51e-13 | 1.33e-04 | 3.65e-12 |

Mode 3

Figure 93: Abaqus — mode 3, 51.969 Hz

Figure 94: Sesam — mode 3, 51.970 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 51.969 | -1.34e-03 | 106622 | 4.50e-14 | 9.00e-10 | 122.03 | 2.12e-07 | 3.00e-05 | 11.047 |
| Sesam | 51.970 | 1.34e-03 | 106628 | 6.59e-14 | 9.47e-24 | 122.03 | -2.57e-07 | -3.08e-12 | -11.047 |

Mode 4

Figure 95: Abaqus — mode 4, 75.145 Hz

Figure 96: Sesam — mode 4, 75.172 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 75.145 | -0.02 | 222925 | 1.24e-16 | 36.88 | 1.80e-09 | 1.11e-08 | -6.073 | 4.24e-05 |
| Sesam | 75.172 | +0.02 | 223083 | 1.13e-25 | 36.87 | 8.58e-25 | -3.36e-13 | 6.072 | -9.26e-13 |

Mode 5

Figure 97: Abaqus — mode 5, 96.001 Hz

Figure 98: Sesam — mode 5, 95.934 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 96.001 | +0.04 | 363843 | 8.08e-12 | 6.17e-09 | 5.02e-12 | -2.84e-06 | -7.85e-05 | 2.24e-06 |
| Sesam | 95.934 | -0.04 | 363330 | 7.77e-26 | 1.06e-09 | 4.95e-24 | -2.79e-13 | 3.26e-05 | 2.22e-12 |

Mode 6

Figure 99: Abaqus — mode 6, 125.207 Hz

Figure 100: Sesam — mode 6, 125.173 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 125.210 | +0.01 | 618895 | 4.62e-16 | 0.02 | 1.76e-10 | -2.15e-08 | -0.138 | -1.33e-05 |
| Sesam | 125.173 | -0.01 | 618555 | 1.13e-26 | 0.02 | 5.47e-23 | 1.06e-13 | -0.144 | 7.40e-12 |

Mode 7

Figure 101: Abaqus — mode 7, 142.773 Hz

Figure 102: Sesam — mode 7, 142.768 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 142.770 | 5.60e-04 | 804737 | 9.41e-15 | 3.54 | 1.08e-09 | -9.70e-08 | 1.880 | 3.28e-05 |
| Sesam | 142.768 | -5.60e-04 | 804681 | 3.82e-26 | 3.42 | 8.44e-24 | -1.95e-13 | -1.850 | 2.90e-12 |

Mode 8

Figure 103: Abaqus — mode 8, 166.598 Hz

Figure 104: Sesam — mode 8, 166.334 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 166.600 | +0.08 | 1.09572e+06 | 1.06e-14 | 5.52 | 1.13e-08 | -1.03e-07 | 2.349 | 1.06e-04 |
| Sesam | 166.334 | -0.08 | 1.09224e+06 | 4.59e-26 | 5.56 | 5.51e-24 | 2.14e-13 | -2.358 | -2.35e-12 |

Mode 9

Figure 105: Abaqus — mode 9, 192.840 Hz

Figure 106: Sesam — mode 9, 193.308 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 192.840 | -0.12 | 1.4681e+06 | 5.91e-14 | 0.67 | 5.79e-09 | 2.43e-07 | -0.818 | -7.61e-05 |
| Sesam | 193.308 | +0.12 | 1.47524e+06 | 3.41e-27 | 0.64 | 9.39e-26 | 5.84e-14 | -0.801 | 3.06e-13 |

Mode 10

Figure 107: Abaqus — mode 10, 233.355 Hz

Figure 108: Sesam — mode 10, 234.613 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 233.360 | -0.27 | 2.14979e+06 | 3.28e-15 | 1.38 | 3.26e-07 | -5.73e-08 | 1.175 | 0.001 |
| Sesam | 234.613 | +0.27 | 2.17302e+06 | 1.23e-18 | 1.40 | 2.13e-20 | -1.11e-09 | -1.185 | -1.46e-10 |

Mode 11

Figure 109: Abaqus — mode 11, 250.125 Hz

Figure 110: Sesam — mode 11, 250.240 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 250.130 | -0.02 | 2.46987e+06 | 9.03e-11 | 3.18e-08 | 4.03e-09 | 9.50e-06 | 1.78e-04 | 6.35e-05 |
| Sesam | 250.240 | +0.02 | 2.47215e+06 | 1.60e-13 | 1.65e-06 | 2.77e-15 | -4.01e-07 | 0.001 | -5.26e-08 |

Shell, 2nd order, QUAD

Mode 1

Figure 111: Abaqus — mode 1, 12.758 Hz

Figure 112: Sesam — mode 1, 12.755 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.758 | +0.01 | 6426.1 | 1.23e-25 | 119.92 | 9.18e-25 | 3.51e-13 | 10.951 | 9.58e-13 |
| Sesam | 12.755 | -0.01 | 6423.15 | 3.36e-26 | 119.94 | 2.47e-24 | -1.83e-13 | -10.952 | -1.57e-12 |

Mode 2

Figure 113: Abaqus — mode 2, 19.486 Hz

Figure 114: Sesam — mode 2, 19.473 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 19.486 | +0.03 | 14990 | 1.04e-26 | 4.65e-20 | 2.40e-23 | 1.02e-13 | 2.16e-10 | 4.90e-12 |
| Sesam | 19.473 | -0.03 | 14970.3 | 4.30e-26 | 1.24e-18 | 1.37e-23 | -2.07e-13 | 1.11e-09 | 3.71e-12 |

Mode 3

Figure 115: Abaqus — mode 3, 51.998 Hz

Figure 116: Sesam — mode 3, 51.970 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 51.998 | +0.03 | 106740 | 5.15e-27 | 9.66e-26 | 121.97 | 7.18e-14 | 3.11e-13 | 11.044 |
| Sesam | 51.970 | -0.03 | 106628 | 6.42e-26 | 7.11e-25 | 122.03 | -2.53e-13 | -8.43e-13 | 11.047 |

Mode 4

Figure 117: Abaqus — mode 4, 75.149 Hz

Figure 118: Sesam — mode 4, 75.125 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 75.149 | +0.02 | 222949 | 1.40e-27 | 36.85 | 6.29e-24 | -3.75e-14 | -6.071 | 2.51e-12 |
| Sesam | 75.125 | -0.02 | 222804 | 2.91e-28 | 36.86 | 2.56e-24 | -1.70e-14 | -6.071 | 1.60e-12 |

Mode 5

Figure 119: Abaqus — mode 5, 95.920 Hz

Figure 120: Sesam — mode 5, 95.889 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 95.920 | +0.02 | 363224 | 7.50e-27 | 2.54e-21 | 8.04e-25 | -8.66e-14 | -5.04e-11 | -8.97e-13 |
| Sesam | 95.889 | -0.02 | 362989 | 1.39e-26 | 1.97e-21 | 1.90e-24 | 1.18e-13 | 4.44e-11 | -1.38e-12 |

Mode 6

Figure 121: Abaqus — mode 6, 125.001 Hz

Figure 122: Sesam — mode 6, 124.987 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 125.000 | +0.01 | 616856 | 5.44e-26 | 0.02 | 4.78e-23 | -2.33e-13 | -0.137 | 6.92e-12 |
| Sesam | 124.987 | -0.01 | 616722 | 1.29e-26 | 0.02 | 6.86e-24 | -1.13e-13 | -0.137 | 2.62e-12 |

Mode 7

Figure 123: Abaqus — mode 7, 142.325 Hz

Figure 124: Sesam — mode 7, 142.234 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 142.320 | +0.03 | 799690 | 2.49e-27 | 3.45 | 4.42e-24 | -4.99e-14 | 1.858 | -2.10e-12 |
| Sesam | 142.234 | -0.03 | 798665 | 6.03e-28 | 3.45 | 4.89e-25 | 2.46e-14 | 1.859 | -6.99e-13 |

Mode 8

Figure 125: Abaqus — mode 8, 165.849 Hz

Figure 126: Sesam — mode 8, 165.718 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 165.850 | +0.04 | 1.08589e+06 | 2.43e-27 | 5.53 | 1.06e-23 | -4.93e-14 | 2.351 | 3.25e-12 |
| Sesam | 165.718 | -0.04 | 1.08418e+06 | 4.51e-28 | 5.52 | 2.31e-24 | -2.12e-14 | -2.350 | -1.52e-12 |

Mode 9

Figure 127: Abaqus — mode 9, 192.184 Hz

Figure 128: Sesam — mode 9, 191.999 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 192.180 | +0.05 | 1.45812e+06 | 1.15e-27 | 0.65 | 1.20e-25 | -3.40e-14 | -0.808 | 3.47e-13 |
| Sesam | 191.999 | -0.05 | 1.45531e+06 | 3.95e-28 | 0.65 | 4.22e-27 | 1.99e-14 | -0.809 | 6.50e-14 |

Mode 10

Figure 129: Abaqus — mode 10, 232.867 Hz

Figure 130: Sesam — mode 10, 232.626 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 232.870 | +0.05 | 2.14079e+06 | 4.88e-26 | 1.37 | 1.33e-23 | -2.21e-13 | 1.170 | -3.65e-12 |
| Sesam | 232.626 | -0.05 | 2.13637e+06 | 1.30e-26 | 1.37 | 2.70e-25 | -1.14e-13 | -1.171 | -5.20e-13 |

Mode 11

Figure 131: Abaqus — mode 11, 250.173 Hz

Figure 132: Sesam — mode 11, 250.095 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 250.170 | +0.01 | 2.47082e+06 | 3.28e-21 | 3.98e-19 | 6.25e-19 | 5.72e-11 | 6.31e-10 | -7.90e-10 |
| Sesam | 250.095 | -0.01 | 2.46928e+06 | 1.10e-19 | 3.85e-22 | 2.18e-21 | -3.32e-10 | -1.96e-11 | 4.67e-11 |

Shell, 2nd order, QUAD, reduced integration

Mode 1

Figure 133: Abaqus — mode 1, 12.758 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.758 | 6426.1 | 1.23e-25 | 119.92 | 9.18e-25 | 3.51e-13 | 10.951 | 9.58e-13 |

Mode 2

Figure 134: Abaqus — mode 2, 19.486 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 19.486 | 14990 | 1.04e-26 | 4.65e-20 | 2.40e-23 | 1.02e-13 | 2.16e-10 | 4.90e-12 |

Mode 3

Figure 135: Abaqus — mode 3, 51.998 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 51.998 | 106740 | 5.15e-27 | 9.66e-26 | 121.97 | 7.18e-14 | 3.11e-13 | 11.044 |

Mode 4

Figure 136: Abaqus — mode 4, 75.149 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 75.149 | 222949 | 1.40e-27 | 36.85 | 6.29e-24 | -3.75e-14 | -6.071 | 2.51e-12 |

Mode 5

Figure 137: Abaqus — mode 5, 95.920 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 95.920 | 363224 | 7.50e-27 | 2.54e-21 | 8.04e-25 | -8.66e-14 | -5.04e-11 | -8.97e-13 |

Mode 6

Figure 138: Abaqus — mode 6, 125.001 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 125.000 | 616856 | 5.44e-26 | 0.02 | 4.78e-23 | -2.33e-13 | -0.137 | 6.92e-12 |

Mode 7

Figure 139: Abaqus — mode 7, 142.325 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 142.320 | 799690 | 2.49e-27 | 3.45 | 4.42e-24 | -4.99e-14 | 1.858 | -2.10e-12 |

Mode 8

Figure 140: Abaqus — mode 8, 165.849 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 165.850 | 1.08589e+06 | 2.43e-27 | 5.53 | 1.06e-23 | -4.93e-14 | 2.351 | 3.25e-12 |

Mode 9

Figure 141: Abaqus — mode 9, 192.184 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 192.180 | 1.45812e+06 | 1.15e-27 | 0.65 | 1.20e-25 | -3.40e-14 | -0.808 | 3.47e-13 |

Mode 10

Figure 142: Abaqus — mode 10, 232.867 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 232.870 | 2.14079e+06 | 4.88e-26 | 1.37 | 1.33e-23 | -2.21e-13 | 1.170 | -3.65e-12 |

Mode 11

Figure 143: Abaqus — mode 11, 250.173 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 250.170 | 2.47082e+06 | 3.28e-21 | 3.98e-19 | 6.25e-19 | 5.72e-11 | 6.31e-10 | -7.90e-10 |

Solid, 1st order, TET

Mode 1

Figure 144: Abaqus — mode 1, 15.790 Hz

Figure 145: Sesam — mode 1, 15.793 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 15.790 | -0.01 | 9842.9 | 2.50e-14 | 117.47 | 2.82e-04 | 1.58e-07 | 10.838 | -0.017 |
| Sesam | 15.793 | +0.01 | 9846.92 | 5.82e-13 | 117.52 | 2.82e-04 | 7.63e-07 | -10.841 | 0.017 |

Mode 2

Figure 146: Abaqus — mode 2, 51.532 Hz

Figure 147: Sesam — mode 2, 51.542 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 51.532 | -0.01 | 104836 | 1.44e-08 | 7.90e-04 | 118.25 | 1.20e-04 | 0.028 | 10.874 |
| Sesam | 51.542 | +0.01 | 104877 | 1.53e-08 | 7.86e-04 | 118.29 | -1.24e-04 | -0.028 | -10.876 |

Mode 3

Figure 148: Abaqus — mode 3, 97.588 Hz

Figure 149: Sesam — mode 3, 97.731 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 97.588 | -0.07 | 375967 | 4.61e-08 | 34.91 | 3.71e-04 | -2.15e-04 | -5.909 | 0.019 |
| Sesam | 97.731 | +0.07 | 377075 | 4.30e-08 | 34.94 | 3.65e-04 | 2.07e-04 | 5.911 | -0.019 |

Mode 4

Figure 150: Abaqus — mode 4, 104.803 Hz

Figure 151: Sesam — mode 4, 105.691 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 104.800 | -0.42 | 433617 | 6.83e-06 | 0.01 | 1.07e-05 | 0.003 | -0.075 | 0.003 |
| Sesam | 105.691 | +0.42 | 440993 | 6.99e-06 | 4.82e-03 | 1.08e-05 | -0.003 | 0.069 | -0.003 |

Mode 5

Figure 152: Abaqus — mode 5, 260.590 Hz

Figure 153: Sesam — mode 5, 260.887 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 260.590 | -0.06 | 2.68086e+06 | 3.30e-07 | 0.02 | 39.75 | -0.001 | 0.132 | -6.305 |
| Sesam | 260.887 | +0.06 | 2.68699e+06 | 3.61e-07 | 0.01 | 39.76 | -0.001 | 0.122 | -6.305 |

Mode 6

Figure 154: Abaqus — mode 6, 266.263 Hz

Figure 155: Sesam — mode 6, 267.131 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 266.260 | -0.16 | 2.79886e+06 | 1.72e-08 | 13.24 | 0.05 | 1.31e-04 | 3.638 | 0.226 |
| Sesam | 267.131 | +0.16 | 2.81714e+06 | 1.49e-08 | 13.26 | 0.04 | 1.22e-04 | 3.642 | 0.207 |

Mode 7

Figure 156: Abaqus — mode 7, 285.051 Hz

Figure 157: Sesam — mode 7, 287.461 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 285.050 | -0.42 | 3.20779e+06 | 1.44e-07 | 9.28e-04 | 1.25e-03 | 3.79e-04 | 0.030 | -0.035 |
| Sesam | 287.461 | +0.42 | 3.26225e+06 | 1.26e-07 | 7.47e-04 | 1.07e-03 | 3.54e-04 | 0.027 | -0.033 |

Mode 8

Figure 158: Abaqus — mode 8, 432.310 Hz

Figure 159: Sesam — mode 8, 432.347 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 432.310 | -4.31e-03 | 7.37819e+06 | 153.38 | 2.06e-06 | 3.31e-08 | 12.385 | -0.001 | 1.82e-04 |
| Sesam | 432.347 | 4.31e-03 | 7.37947e+06 | 153.37 | 1.91e-06 | 2.88e-08 | 12.384 | -0.001 | 1.70e-04 |

Mode 9

Figure 160: Abaqus — mode 9, 500.864 Hz

Figure 161: Sesam — mode 9, 503.862 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 500.860 | -0.30 | 9.90373e+06 | 3.39e-05 | 6.33 | 4.73e-05 | -0.006 | -2.515 | 0.007 |
| Sesam | 503.862 | +0.30 | 1.00226e+07 | 3.01e-05 | 6.34 | 4.92e-05 | -0.005 | -2.518 | 0.007 |

Mode 10

Figure 162: Abaqus — mode 10, 534.263 Hz

Figure 163: Sesam — mode 10, 539.506 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 534.260 | -0.49 | 1.12686e+07 | 2.60e-04 | 2.77e-04 | 6.63e-06 | -0.016 | -0.017 | 0.003 |
| Sesam | 539.506 | +0.49 | 1.14909e+07 | 2.36e-04 | 2.31e-04 | 6.55e-06 | -0.015 | -0.015 | 0.003 |

Mode 11

Figure 164: Abaqus — mode 11, 596.780 Hz

Figure 165: Sesam — mode 11, 598.200 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 596.780 | -0.12 | 1.40601e+07 | 4.55e-07 | 1.33e-07 | 12.80 | -0.001 | -3.65e-04 | 3.578 |
| Sesam | 598.200 | +0.12 | 1.41271e+07 | 4.36e-07 | 3.62e-08 | 12.80 | 0.001 | 1.90e-04 | -3.578 |

Solid, 1st order, HEX

Mode 1

Figure 166: Abaqus — mode 1, 13.435 Hz

Figure 167: Sesam — mode 1, 12.977 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 13.435 | +1.74 | 7125.8 | 4.92e-29 | 116.39 | 5.47e-23 | -7.01e-15 | 10.788 | -7.39e-12 |
| Sesam | 12.977 | -1.74 | 6647.9 | 8.06e-25 | 116.43 | 1.16e-20 | -8.98e-13 | 10.790 | 1.08e-10 |

Mode 2

Figure 168: Abaqus — mode 2, 20.731 Hz

Figure 169: Sesam — mode 2, 20.302 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 20.731 | +1.05 | 16967 | 7.03e-26 | 1.48e-18 | 2.87e-22 | 2.65e-13 | 1.22e-09 | -1.69e-11 |
| Sesam | 20.302 | -1.05 | 16271.1 | 2.93e-27 | 2.21e-19 | 3.64e-20 | -5.41e-14 | -4.70e-10 | 1.91e-10 |

Mode 3

Figure 170: Abaqus — mode 3, 50.678 Hz

Figure 171: Sesam — mode 3, 50.618 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 50.678 | +0.06 | 101392 | 1.09e-25 | 2.57e-22 | 118.20 | -3.30e-13 | 1.60e-11 | 10.872 |
| Sesam | 50.618 | -0.06 | 101151 | 1.08e-24 | 8.70e-21 | 118.29 | -1.04e-12 | -9.33e-11 | 10.876 |

Mode 4

Figure 172: Abaqus — mode 4, 82.799 Hz

Figure 173: Sesam — mode 4, 77.392 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 82.799 | +3.38 | 270652 | 7.52e-26 | 36.02 | 2.70e-22 | -2.74e-13 | -6.002 | 1.64e-11 |
| Sesam | 77.392 | -3.38 | 236458 | 6.94e-26 | 35.94 | 6.82e-22 | -2.63e-13 | 5.995 | -2.61e-11 |

Mode 5

Figure 174: Abaqus — mode 5, 100.195 Hz

Figure 175: Sesam — mode 5, 98.119 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 100.190 | +1.04 | 396325 | 4.10e-26 | 6.87e-21 | 7.52e-23 | 2.03e-13 | -8.29e-11 | 8.67e-12 |
| Sesam | 98.119 | -1.04 | 380068 | 1.62e-25 | 2.53e-21 | 7.29e-23 | 4.03e-13 | -5.03e-11 | -8.54e-12 |

Mode 6

Figure 176: Abaqus — mode 6, 223.139 Hz

Figure 177: Sesam — mode 6, 133.016 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 223.140 | +25.30 | 1.96567e+06 | 2.33e-25 | 12.40 | 1.64e-23 | -4.83e-13 | 3.522 | 4.05e-12 |
| Sesam | 133.016 | -25.30 | 698501 | 3.07e-26 | 0.01 | 1.44e-20 | -1.75e-13 | -0.114 | 1.20e-10 |

Mode 7

Figure 178: Abaqus — mode 7, 254.984 Hz

Figure 179: Sesam — mode 7, 153.833 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 254.980 | +24.74 | 2.56676e+06 | 2.24e-25 | 2.85e-24 | 40.01 | -4.74e-13 | 1.69e-12 | -6.326 |
| Sesam | 153.833 | -24.74 | 934235 | 3.75e-25 | 3.20 | 1.37e-21 | -6.13e-13 | -1.788 | 3.70e-11 |

Mode 8

Figure 180: Abaqus — mode 8, 260.095 Hz

Figure 181: Sesam — mode 8, 178.565 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 260.100 | +18.59 | 2.6707e+06 | 2.66e-26 | 6.24e-23 | 3.82e-21 | -1.63e-13 | 7.90e-12 | 6.18e-11 |
| Sesam | 178.565 | -18.59 | 1.25879e+06 | 3.19e-27 | 5.71 | 3.11e-21 | -5.65e-14 | 2.390 | 5.58e-11 |

Mode 9

Figure 182: Abaqus — mode 9, 396.567 Hz

Figure 183: Sesam — mode 9, 214.689 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 396.570 | +29.76 | 6.20858e+06 | 2.52e-22 | 5.69 | 2.13e-23 | 1.59e-11 | -2.385 | 4.61e-12 |
| Sesam | 214.689 | -29.76 | 1.81962e+06 | 5.15e-26 | 0.44 | 2.62e-22 | -2.27e-13 | 0.662 | -1.62e-11 |

Mode 10

Figure 184: Abaqus — mode 10, 431.802 Hz

Figure 185: Sesam — mode 10, 253.463 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 431.800 | +26.02 | 7.36085e+06 | 153.54 | 1.07e-21 | 9.26e-21 | 12.391 | 3.27e-11 | -9.62e-11 |
| Sesam | 253.463 | -26.02 | 2.53623e+06 | 1.48e-19 | 5.14e-20 | 40.27 | -3.84e-10 | 2.27e-10 | -6.346 |

Mode 11

Figure 186: Abaqus — mode 11, 486.965 Hz

Figure 187: Sesam — mode 11, 256.050 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 486.960 | +31.08 | 9.3617e+06 | 1.09e-16 | 2.28e-18 | 1.17e-18 | 1.04e-08 | 1.51e-09 | 1.08e-09 |
| Sesam | 256.050 | -31.08 | 2.58827e+06 | 4.64e-20 | 1.35e-20 | 4.71e-23 | 2.15e-10 | -1.16e-10 | -6.86e-12 |

Solid, 1st order, HEX, reduced integration

Mode 1

Figure 188: Abaqus — mode 1, 12.514 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.514 | 6182 | 1.92e-26 | 116.92 | 5.56e-23 | 1.39e-13 | 10.813 | -7.45e-12 |

Mode 2

Figure 189: Abaqus — mode 2, 34.157 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 34.157 | 46060 | 4.54e-27 | 1.11e-19 | 8.80e-22 | 6.74e-14 | 3.33e-10 | -2.97e-11 |

Mode 3

Figure 190: Abaqus — mode 3, 49.598 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 49.598 | 97114 | 1.57e-26 | 1.07e-20 | 118.97 | -1.25e-13 | 1.03e-10 | 10.907 |

Mode 4

Figure 191: Abaqus — mode 4, 55.271 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 55.271 | 120602 | 1.14e-25 | 2.12 | 1.05e-18 | -3.37e-13 | -1.456 | 1.02e-09 |

Mode 5

Figure 192: Abaqus — mode 5, 64.305 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 64.305 | 163249 | 1.13e-26 | 8.64e-20 | 1.24e-20 | -1.06e-13 | -2.94e-10 | 1.11e-10 |

Mode 6

Figure 193: Abaqus — mode 6, 74.866 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 74.866 | 221271 | 1.12e-26 | 31.40 | 1.02e-20 | 1.06e-13 | -5.603 | -1.01e-10 |

Mode 7

Figure 194: Abaqus — mode 7, 90.033 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 90.033 | 320010 | 1.17e-28 | 2.79 | 5.27e-21 | -1.08e-14 | -1.671 | 7.26e-11 |

Mode 8

Figure 195: Abaqus — mode 8, 124.578 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 124.580 | 612693 | 3.87e-24 | 3.38e-21 | 2.90e-24 | -1.97e-12 | -5.82e-11 | -1.70e-12 |

Mode 9

Figure 196: Abaqus — mode 9, 155.992 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 155.990 | 960645 | 7.43e-21 | 1.12 | 2.07e-20 | -8.62e-11 | -1.056 | -1.44e-10 |

Mode 10

Figure 197: Abaqus — mode 10, 159.952 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 159.950 | 1.01004e+06 | 3.65e-04 | 8.80e-19 | 3.04e-19 | 0.019 | -9.38e-10 | 5.51e-10 |

Mode 11

Figure 198: Abaqus — mode 11, 162.557 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 162.560 | 1.04321e+06 | 4.88e-19 | 1.57e-19 | 3.29e-20 | -6.98e-10 | 3.96e-10 | -1.81e-10 |

Solid, 2nd order, TET

Mode 1

Figure 199: Abaqus — mode 1, 12.954 Hz

Figure 200: Sesam — mode 1, 12.954 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.954 | 1.00e-03 | 6624.5 | 3.65e-13 | 116.51 | 2.81e-11 | 6.04e-07 | 10.794 | 5.30e-06 |
| Sesam | 12.954 | -1.00e-03 | 6624.46 | 3.01e-13 | 116.51 | 2.70e-11 | -5.49e-07 | -10.794 | -5.20e-06 |

Mode 2

Figure 201: Abaqus — mode 2, 20.294 Hz

Figure 202: Sesam — mode 2, 20.294 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 20.294 | -1.13e-03 | 16260 | 8.19e-12 | 5.86e-08 | 1.99e-09 | 2.86e-06 | -2.42e-04 | 4.47e-05 |
| Sesam | 20.294 | 1.13e-03 | 16259.8 | 8.59e-12 | 5.86e-08 | 1.98e-09 | 2.93e-06 | -2.42e-04 | 4.45e-05 |

Mode 3

Figure 203: Abaqus — mode 3, 50.540 Hz

Figure 204: Sesam — mode 3, 50.540 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 50.540 | -4.95e-05 | 100840 | 3.09e-10 | 1.03e-08 | 118.35 | 1.76e-05 | -1.01e-04 | 10.879 |
| Sesam | 50.540 | 4.95e-05 | 100840 | 3.10e-10 | 1.03e-08 | 118.35 | 1.76e-05 | -1.01e-04 | 10.879 |

Mode 4

Figure 205: Abaqus — mode 4, 77.128 Hz

Figure 206: Sesam — mode 4, 77.128 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 77.128 | 1.43e-04 | 234845 | 6.40e-12 | 35.97 | 1.94e-09 | 2.53e-06 | -5.997 | -4.41e-05 |
| Sesam | 77.128 | -1.43e-04 | 234845 | 1.04e-11 | 35.97 | 1.89e-09 | 3.22e-06 | -5.997 | -4.35e-05 |

Mode 5

Figure 207: Abaqus — mode 5, 97.830 Hz

Figure 208: Sesam — mode 5, 97.830 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 97.830 | 2.10e-04 | 377834 | 1.93e-10 | 3.14e-07 | 6.49e-09 | -1.39e-05 | -0.001 | 8.06e-05 |
| Sesam | 97.830 | -2.10e-04 | 377833 | 1.99e-10 | 3.14e-07 | 6.59e-09 | 1.41e-05 | 0.001 | -8.12e-05 |

Mode 6

Figure 209: Abaqus — mode 6, 132.482 Hz

Figure 210: Sesam — mode 6, 132.478 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 132.480 | 6.04e-04 | 692901 | 1.05e-09 | 0.01 | 2.04e-09 | 3.23e-05 | 0.119 | -4.51e-05 |
| Sesam | 132.478 | -6.04e-04 | 692867 | 1.14e-09 | 0.01 | 2.04e-09 | -3.38e-05 | -0.119 | 4.52e-05 |

Mode 7

Figure 211: Abaqus — mode 7, 153.634 Hz

Figure 212: Sesam — mode 7, 153.631 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 153.630 | -4.56e-04 | 931828 | 1.43e-08 | 3.12 | 2.11e-09 | -1.19e-04 | 1.768 | -4.59e-05 |
| Sesam | 153.631 | 4.56e-04 | 931794 | 1.56e-08 | 3.12 | 2.15e-09 | 1.25e-04 | -1.768 | 4.63e-05 |

Mode 8

Figure 213: Abaqus — mode 8, 178.118 Hz

Figure 214: Sesam — mode 8, 178.114 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 178.120 | 1.74e-03 | 1.2525e+06 | 2.65e-10 | 5.79 | 5.99e-10 | -1.63e-05 | 2.405 | -2.45e-05 |
| Sesam | 178.114 | -1.74e-03 | 1.25243e+06 | 3.03e-10 | 5.79 | 6.40e-10 | 1.74e-05 | -2.406 | 2.53e-05 |

Mode 9

Figure 215: Abaqus — mode 9, 215.822 Hz

Figure 216: Sesam — mode 9, 215.815 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 215.820 | 1.20e-03 | 1.83887e+06 | 8.48e-08 | 0.60 | 8.50e-08 | -2.91e-04 | 0.774 | 2.91e-04 |
| Sesam | 215.815 | -1.20e-03 | 1.83875e+06 | 9.03e-08 | 0.60 | 8.47e-08 | 3.00e-04 | -0.774 | -2.91e-04 |

Mode 10

Figure 217: Abaqus — mode 10, 253.203 Hz

Figure 218: Sesam — mode 10, 253.203 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 253.200 | -5.73e-04 | 2.53103e+06 | 8.86e-09 | 1.97e-08 | 40.25 | -9.41e-05 | 1.40e-04 | -6.344 |
| Sesam | 253.203 | 5.73e-04 | 2.53103e+06 | 9.13e-09 | 1.99e-08 | 40.25 | -9.56e-05 | 1.41e-04 | -6.344 |

Mode 11

Figure 219: Abaqus — mode 11, 254.466 Hz

Figure 220: Sesam — mode 11, 254.466 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 254.470 | 8.65e-04 | 2.55635e+06 | 1.63e-09 | 6.96e-07 | 4.16e-05 | -4.04e-05 | -0.001 | -0.006 |
| Sesam | 254.466 | -8.65e-04 | 2.55634e+06 | 1.66e-09 | 6.95e-07 | 4.17e-05 | -4.07e-05 | -0.001 | -0.006 |

Solid, 2nd order, HEX

Mode 1

Figure 221: Abaqus — mode 1, 12.955 Hz

Figure 222: Sesam — mode 1, 12.954 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.955 | 3.32e-03 | 6625.8 | 3.08e-24 | 116.50 | 5.64e-20 | 1.76e-12 | 10.794 | 2.37e-10 |
| Sesam | 12.954 | -3.32e-03 | 6624.86 | 3.50e-24 | 116.51 | 1.44e-22 | 1.87e-12 | 10.794 | 1.20e-11 |

Mode 2

Figure 223: Abaqus — mode 2, 20.150 Hz

Figure 224: Sesam — mode 2, 20.044 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 20.150 | +0.26 | 16030 | 2.26e-25 | 4.71e-16 | 2.32e-18 | -4.76e-13 | 2.17e-08 | 1.52e-09 |
| Sesam | 20.044 | -0.26 | 15861.7 | 2.99e-24 | 4.86e-15 | 1.02e-18 | 1.73e-12 | -6.97e-08 | 1.01e-09 |

Mode 3

Figure 225: Abaqus — mode 3, 50.541 Hz

Figure 226: Sesam — mode 3, 50.534 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 50.541 | +0.01 | 100844 | 1.71e-23 | 8.62e-20 | 118.36 | -4.13e-12 | -2.94e-10 | 10.879 |
| Sesam | 50.534 | -0.01 | 100816 | 4.30e-24 | 5.72e-22 | 118.37 | 2.07e-12 | 2.39e-11 | 10.880 |

Mode 4

Figure 227: Abaqus — mode 4, 76.890 Hz

Figure 228: Sesam — mode 4, 76.747 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 76.890 | +0.09 | 233401 | 9.37e-28 | 35.91 | 1.14e-20 | -3.06e-14 | -5.993 | -1.07e-10 |
| Sesam | 76.747 | -0.09 | 232530 | 1.68e-24 | 35.89 | 3.76e-21 | -1.29e-12 | -5.991 | 6.13e-11 |

Mode 5

Figure 229: Abaqus — mode 5, 97.594 Hz

Figure 230: Sesam — mode 5, 97.472 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 97.594 | +0.06 | 376015 | 1.21e-24 | 3.12e-19 | 1.62e-21 | 1.10e-12 | -5.59e-10 | -4.02e-11 |
| Sesam | 97.472 | -0.06 | 375078 | 8.04e-25 | 3.55e-18 | 2.80e-21 | -8.97e-13 | 1.88e-09 | -5.29e-11 |

Mode 6

Figure 231: Abaqus — mode 6, 131.365 Hz

Figure 232: Sesam — mode 6, 130.865 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 131.370 | +0.19 | 681273 | 2.05e-24 | 0.01 | 1.33e-19 | 1.43e-12 | -0.103 | -3.65e-10 |
| Sesam | 130.865 | -0.19 | 676090 | 1.00e-25 | 0.01 | 9.22e-22 | 3.16e-13 | -0.096 | 3.04e-11 |

Mode 7

Figure 233: Abaqus — mode 7, 149.958 Hz

Figure 234: Sesam — mode 7, 148.496 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 149.960 | +0.49 | 887766 | 2.76e-24 | 3.31 | 8.83e-21 | 1.66e-12 | 1.818 | 9.40e-11 |
| Sesam | 148.496 | -0.49 | 870539 | 3.33e-25 | 3.38 | 1.39e-22 | 5.77e-13 | -1.839 | -1.18e-11 |

Mode 8

Figure 235: Abaqus — mode 8, 173.925 Hz

Figure 236: Sesam — mode 8, 172.243 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 173.920 | +0.48 | 1.19422e+06 | 4.36e-26 | 5.42 | 1.76e-20 | 2.09e-13 | 2.328 | -1.33e-10 |
| Sesam | 172.243 | -0.48 | 1.17124e+06 | 8.12e-26 | 5.31 | 3.08e-22 | 2.85e-13 | -2.305 | -1.76e-11 |

Mode 9

Figure 237: Abaqus — mode 9, 205.292 Hz

Figure 238: Sesam — mode 9, 201.904 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 205.290 | +0.83 | 1.66381e+06 | 4.78e-27 | 0.52 | 2.91e-22 | 6.91e-14 | -0.719 | 1.71e-11 |
| Sesam | 201.904 | -0.83 | 1.60935e+06 | 1.55e-24 | 0.56 | 1.71e-22 | -1.25e-12 | -0.746 | 1.31e-11 |

Mode 10

Figure 239: Abaqus — mode 10, 251.347 Hz

Figure 240: Sesam — mode 10, 246.353 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 251.350 | +1.00 | 2.49407e+06 | 1.08e-21 | 1.60 | 3.13e-19 | 3.29e-11 | 1.267 | -5.59e-10 |
| Sesam | 246.353 | -1.00 | 2.39594e+06 | 1.32e-18 | 1.47 | 6.50e-19 | -1.15e-09 | 1.211 | 8.06e-10 |

Mode 11

Figure 241: Abaqus — mode 11, 252.917 Hz

Figure 242: Sesam — mode 11, 252.742 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 252.920 | +0.04 | 2.52532e+06 | 4.22e-22 | 4.70e-20 | 40.25 | 2.05e-11 | 2.17e-10 | -6.344 |
| Sesam | 252.742 | -0.04 | 2.52182e+06 | 2.26e-15 | 2.80e-16 | 40.28 | -4.75e-08 | 1.67e-08 | 6.346 |

Solid, 2nd order, HEX, reduced integration

Mode 1

Figure 243: Abaqus — mode 1, 12.943 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 12.943 | 6613.2 | 5.11e-26 | 116.56 | 2.93e-20 | -2.26e-13 | 10.796 | 1.71e-10 |

Mode 2

Figure 244: Abaqus — mode 2, 20.029 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 20.029 | 15837 | 2.00e-26 | 8.47e-16 | 7.79e-20 | -1.41e-13 | 2.91e-08 | -2.79e-10 |

Mode 3

Figure 245: Abaqus — mode 3, 50.498 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 50.498 | 100671 | 1.64e-24 | 3.78e-20 | 118.41 | -1.28e-12 | -1.94e-10 | 10.882 |

Mode 4

Figure 246: Abaqus — mode 4, 76.687 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 76.687 | 232171 | 2.11e-25 | 35.90 | 8.07e-22 | -4.60e-13 | -5.992 | -2.84e-11 |

Mode 5

Figure 247: Abaqus — mode 5, 97.391 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 97.391 | 374455 | 1.68e-25 | 8.25e-19 | 2.52e-24 | 4.10e-13 | -9.08e-10 | -1.59e-12 |

Mode 6

Figure 248: Abaqus — mode 6, 130.809 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 130.810 | 675515 | 5.64e-26 | 0.01 | 6.07e-21 | -2.37e-13 | -0.097 | 7.79e-11 |

Mode 7

Figure 249: Abaqus — mode 7, 148.474 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 148.470 | 870287 | 1.93e-24 | 3.39 | 2.12e-22 | -1.39e-12 | 1.840 | -1.46e-11 |

Mode 8

Figure 250: Abaqus — mode 8, 172.171 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 172.170 | 1.17026e+06 | 1.27e-25 | 5.31 | 9.73e-21 | 3.57e-13 | 2.304 | 9.86e-11 |

Mode 9

Figure 251: Abaqus — mode 9, 201.839 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 201.840 | 1.60831e+06 | 9.86e-25 | 0.56 | 1.17e-21 | -9.93e-13 | -0.747 | 3.42e-11 |

Mode 10

Figure 252: Abaqus — mode 10, 246.165 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 246.170 | 2.39228e+06 | 3.32e-21 | 1.47 | 1.28e-24 | 5.76e-11 | 1.212 | -1.13e-12 |

Mode 11

Figure 253: Abaqus — mode 11, 252.573 Hz

| Solver | f [Hz] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 252.570 | 2.51846e+06 | 4.57e-21 | 2.25e-19 | 40.26 | 6.76e-11 | 4.75e-10 | -6.345 |

Plate eigenvalue analysis

The plain and the stiffened strip, at the eigen case’s 0.0625 m seed.

Shell, 1st order, plain

Mode 1

Figure 254: Abaqus — mode 1, 1.537 Hz

Figure 255: Sesam — mode 1, 1.537 Hz

| Solver | f [Hz] | Δf vs closed form [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 1.537 | +0.03 | 93.269 | 6.40e-31 | 3.03e-35 | 127.26 | -8.00e-16 | 5.50e-18 | 11.281 |
| Sesam | 1.537 | -4.98e-06 | 93.2152 | 2.62e-40 | 5.38e-45 | 127.17 | -1.62e-20 | 7.33e-23 | -11.277 |
| Closed form | 1.537 |  |  |  |  |  |  |  |  |

Mode 2

Figure 256: Abaqus — mode 2, 6.154 Hz

Figure 257: Sesam — mode 2, 6.146 Hz

| Solver | f [Hz] | Δf vs closed form [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 6.154 | +0.12 | 1494.9 | 5.42e-31 | 1.34e-33 | 5.32e-21 | 7.36e-16 | -3.66e-17 | 7.29e-11 |
| Sesam | 6.146 | 1.13e-05 | 1491.44 | 2.17e-34 | 3.37e-39 | 9.71e-23 | -1.47e-17 | 5.81e-20 | -9.85e-12 |
| Closed form | 6.146 |  |  |  |  |  |  |  |  |

Mode 3

Figure 258: Abaqus — mode 3, 13.866 Hz

Figure 259: Sesam — mode 3, 13.829 Hz

| Solver | f [Hz] | Δf vs closed form [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 13.866 | +0.26 | 7589.9 | 4.43e-30 | 5.80e-32 | 14.14 | 2.10e-15 | -2.41e-16 | -3.760 |
| Sesam | 13.829 | 6.01e-05 | 7550.43 | 2.14e-34 | 3.80e-39 | 14.05 | -1.46e-17 | 6.17e-20 | -3.748 |
| Closed form | 13.829 |  |  |  |  |  |  |  |  |

Mode 4

Figure 260: Abaqus — mode 4, 24.700 Hz

Figure 261: Sesam — mode 4, 24.586 Hz

| Solver | f [Hz] | Δf vs closed form [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 24.700 | +0.46 | 24086 | 1.04e-28 | 7.36e-32 | 1.83e-23 | -1.02e-14 | 2.71e-16 | 4.27e-12 |
| Sesam | 24.586 | 1.01e-04 | 23863.1 | 1.77e-33 | 1.52e-36 | 1.13e-23 | 4.21e-17 | 1.23e-18 | 3.36e-12 |
| Closed form | 24.586 |  |  |  |  |  |  |  |  |

Mode 5

Figure 262: Abaqus — mode 5, 38.695 Hz

Figure 263: Sesam — mode 5, 38.415 Hz

| Solver | f [Hz] | Δf vs closed form [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 38.695 | +0.73 | 59111 | 1.02e-28 | 1.84e-30 | 5.09 | 1.01e-14 | -1.36e-15 | 2.256 |
| Sesam | 38.415 | 2.42e-04 | 58259.8 | 1.78e-26 | 3.30e-27 | 5.00 | 1.33e-13 | 5.75e-14 | 2.236 |
| Closed form | 38.415 |  |  |  |  |  |  |  |  |

Mode 6

Figure 264: Abaqus — mode 6, 55.900 Hz

Figure 265: Sesam — mode 6, 55.318 Hz

| Solver | f [Hz] | Δf vs closed form [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 55.900 | +1.05 | 123361 | 4.94e-20 | 7.80e-26 | 3.09e-20 | -2.22e-10 | -2.79e-13 | -1.76e-10 |
| Sesam | 55.318 | 5.30e-04 | 120808 | 1.43e-17 | 2.77e-18 | 6.63e-16 | 3.78e-09 | 1.66e-09 | -2.58e-08 |
| Closed form | 55.318 |  |  |  |  |  |  |  |  |

Shell, 1st order, stiffened

Mode 1

Figure 266: Abaqus — mode 1, 21.242 Hz

Figure 267: Sesam — mode 1, 18.917 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 21.242 | +5.79 | 17813 | 0.05 | 9.45e-26 | 195.80 | -0.214 | -3.07e-13 | 13.993 |
| Sesam | 18.917 | -5.79 | 14127.2 | 2.29e-32 | 2.10e-37 | 195.94 | -1.51e-16 | -4.58e-19 | 13.998 |

Mode 2

Figure 268: Abaqus — mode 2, 80.469 Hz

Figure 269: Sesam — mode 2, 70.063 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 80.469 | +6.91 | 255632 | 0.16 | 1.91e-25 | 3.42e-05 | 0.403 | 4.37e-13 | 0.006 |
| Sesam | 70.063 | -6.91 | 193794 | 4.69e-34 | 9.89e-36 | 8.76e-26 | 2.17e-17 | 3.14e-18 | 2.96e-13 |

Mode 3

Figure 270: Abaqus — mode 3, 93.088 Hz

Figure 271: Sesam — mode 3, 92.575 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 93.088 | +0.28 | 342097 | 2.75e-24 | 3.95 | 1.03e-23 | 1.66e-12 | 1.988 | 3.22e-12 |
| Sesam | 92.575 | -0.28 | 338332 | 5.73e-33 | 2.02e-07 | 3.30e-28 | -7.57e-17 | 4.49e-04 | 1.82e-14 |

Mode 4

Figure 272: Abaqus — mode 4, 106.618 Hz

Figure 273: Sesam — mode 4, 100.998 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 106.620 | +2.71 | 448765 | 1.14e-22 | 2.35e-08 | 5.91e-22 | 1.07e-11 | 1.53e-04 | -2.43e-11 |
| Sesam | 100.998 | -2.71 | 402700 | 1.38e-29 | 1.02e-09 | 1.26e-26 | -3.71e-15 | 3.19e-05 | 1.12e-13 |

Mode 5

Figure 274: Abaqus — mode 5, 128.504 Hz

Figure 275: Sesam — mode 5, 115.222 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 128.500 | +5.45 | 651919 | 5.66e-20 | 0.39 | 1.44e-19 | 2.38e-10 | -0.624 | 3.79e-10 |
| Sesam | 115.222 | -5.45 | 524118 | 4.47e-23 | 4.62e-07 | 1.89e-22 | -6.69e-12 | -0.001 | 1.37e-11 |

Mode 6

Figure 276: Abaqus — mode 6, 155.525 Hz

Figure 277: Sesam — mode 6, 134.344 Hz

| Solver | f [Hz] | Δf vs mean [%] | λ [rad²/s²] | Meff X [kg] | Meff Y [kg] | Meff Z [kg] | Γ X | Γ Y | Γ Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abaqus | 155.520 | +7.31 | 954899 | 0.26 | 3.11e-16 | 20.18 | 0.507 | 1.76e-08 | -4.492 |
| Sesam | 134.344 | -7.31 | 712516 | 3.13e-16 | 2.60e-18 | 20.78 | -1.77e-08 | 1.61e-09 | 4.558 |

Plate static analysis

The deflected strip under 1 kPa, plain and stiffened, at the same 0.0625 m seed. The other seeds deform into the same shape; their convergence is in the summary table.

Shell, 1st order, plain, seed 0.0625 m

Figure 278: Abaqus — displacement

Figure 279: Sesam — displacement

| Solver | Mid-span u_z [mm] | Δ vs closed form [%] | Peak u_z [mm] | Peak u_x [mm] | Peak u_y [mm] |
| --- | --- | --- | --- | --- | --- |
| Abaqus | 173.2686 | -0.04 | 173.2686 | 5.59e-17 | 4.12e-29 |
| Sesam | 173.2995 | -0.02 | 173.2995 | 0.0000 | 0.0000 |
| Closed form | 173.3333 |  |  |  |  |

Shell, 1st order, stiffened, seed 0.0625 m

Figure 280: Abaqus — displacement

Figure 281: Sesam — displacement

| Solver | Mid-span u_z [mm] | Δ vs mean [%] | Peak u_z [mm] | Peak u_x [mm] | Peak u_y [mm] |
| --- | --- | --- | --- | --- | --- |
| Abaqus | 0.5855 | -11.59 | 0.5934 | 0.0118 | 2.12e-05 |
| Sesam | 0.7390 | +11.59 | 0.7474 | 1.25e-17 | 2.34e-17 |